Video Electronics Standards Association               VBE/Core 2.0 Standard

2150 North First Street, Suite 440

San Jose, CA 95131-2029

Phone: (408) 435-0333

FAX: (408) 435-8225

VESA BIOS EXTENSION (VBE)

Core Functions

Version: 2.0

NOTE: This is _NOT_ an official version of the VBE/Core 2.0 standard,

but is provided as is by SciTech Software until the official

specification is made available electronically. This document

contains a summary of the important portions of the standard

related to developing working code for VBE 2.0.

For more information on the official specification, please

contact VESA at the address listed above.

1.    Introduction

This document contains the VESA BIOS Extension (VBE) specification for

standard software access to graphics display controllers that support

resolutions, color depths, and frame buffer organizations beyond the

VGA hardware standard.  It is intended for use by both applications

programmers and system software developers.

System software developers may use this document to supplement the

System and INT 10h ROM BIOS functions to provide the VBE services.

Application developers can use this document as a guide to programming

all VBE compatible devices.

To understand the VBE specification, some knowledge of 80x86 assembly

language and the VGA hardware registers may be required.  However,

the services described in this specification may be called from any

high-level programming language that provides a mechanism for

generating software interrupts with the 80x86 registers set to user-

specified values.

In this specification, 'VBE' and 'VBE 2.0' are synonymous with 'VBE

Core Functions version 2.0'.

1.1. Scope of the VBE Standard

The primary purpose of the VESA VBE  is to provide standard software

support for the many unique implementations of Super VGA (SVGA)

graphics controllers on the PC platform that provide features beyond

the original VGA hardware standard.  This is to provide a feasible

mechanism by which application developers can take advantage of this

nonstandard hardware in graphics applications.

The VBE specification offers an extensible software foundation which

allows it to evolve as display and audio devices evolve over time,

without sacrificing backward software compatibility with older

implementations.  New application software should be able to work with

older hardware, and application software that has already shipped

should work correctly on new hardware devices.

VBE services provide standard access to all resolutions and color

depths provided on the display controller, and report the availability

and details of all supported configurations to the application as

necessary.

VBE implementations facilitate the field support of audio and display

hardware by providing the application software with the manufacturer's

name and the product identification of the display hardware.

Since graphics controller services on the PC are typically implemented

in ROM, the VBE services are defined so that they should be

implemented within the standard VGA ROM.  When ROM implementations of

VBE are not possible, or when field software upgrades to the onboard

ROM are necessary, the VBE implementation may be also offered as a

device driver or DOS Terminate & Stay Resident (TSR) program.

The standard VBE functions may be supplemented by OEM's as necessary

to support custom or proprietary functions unique to the manufacturer.

This mechanism enables the OEM to establish functions that may be

standard to the product line, or provide access to special hardware

enhancements.

Although previous VBE standards assumed that the underlying graphics

architecture was a VGA device, the display services described by VBE

2.0 can be implemented on any frame buffer oriented graphics device.

The majority of VBE services facilitate the setup and configuration of

the hardware, allowing applications high performance, direct access to

the configured device at runtime.  To further improve the performance

of flat frame buffer display devices in extended resolutions, VBE 2.0

provides new memory models that do not require the traditional frame

buffer "banking" mechanisms.

VBE is expected to work on all 80x86 platforms, in real and protected

modes.

Since some modern display devices are designed without any VGA

support, two display controllers may be present in the system.  One

display controller would be used for VGA compatibility, and the other

used for graphic extensions to the basic VGA modes, resolutions, and

frame buffer models.  Therefore, VBE must be able offer the

application automatic access to the appropriate device based on the

mode or resolution that is requested by the application.

Currently beyond the scope of the VBE specification is the handling of

hardware configuration and installation issues.  It was originally

considered to become part of VBE 2.0, however we have deferred the

issues to the Graphics Configuration Supplemental Specification.  In

addition, it is also possible for an OEM to define their own

extensions using the OEM Supplemental Specification if required.

1.2. Backgrounder

1

The IBM VGA  has become a de facto standard in the PC graphics world.

A multitude of different VGA offerings exist in the marketplace, each

one providing BIOS or register compatibility with the IBM VGA. More

and more of these VGA compatible products implement various supersets

of the VGA standard. These extensions range from higher resolutions

and more colors to improved performance and even some graphics

processing capabilities. Intense competition has dramatically improved

the price/performance ratio, to the benefit of the end user.

However, several serious problems face a software developer who

intends to take advantage of these "Super VGA"  environments. Because

there is no standard hardware implementation, the developer is faced

with widely disparate Super VGA hardware architecture. Lacking a

common software interface, designing applications for these

environments is costly and technically difficult. Except for

applications supported by OEM-specific display drivers, very few

software packages can take advantage of the power and capabilities of

Super VGA products.

The VBE standard was originally conceived to enable the development of

applications that wished to take advantage of display resolutions and

color depths beyond the VGA definition.  The need for an application

or software standard was recognized by the developers of graphic

hardware to encourage the use and acceptance of their rapidly

advancing product families.  It became obvious that the majority of

software application developers did not have the resources to develop

and support custom device level software for the hundreds of display

boards on the market.  Therefore the rich new features of these

display devices were not being used outside of the relatively small

CAD market, and only then after considerable effort.

Indeed, the need for a standard for SVGA display adapters became so

important that the VESA organization was formed to seek out a

solution.  The original VBE standard was devised and agreed upon by

each of the active display controller manufacturers, and has since

been adopted by DOS application developers to enable use of non-VGA

extended display modes.

As time went along VBE 1.1 was created to add more video modes and

increased logical line length/double buffering support.  VBE 1.2 was

created to add modes and also added high color RAMDAC support.

In the three years since VBE 1.2 was approved we have seen the

standard become widely accepted and many successful programs have

embraced VBE.  However, it has become obvious that the need for a more

robust and extensible standard exists.  Early extensions to the VGA

standard continued using all of the original VGA I/O ports and frame

buffer address to communicate with the controller hardware.  As we've

seen, the supported resolutions and color depths have grown,

intelligent controllers with BITBLT and LineDraw Functions have become

common, and new flat frame buffer memory models have appeared along

with display controllers that are not based on VGA in any way.  VBE

2.0 and future extensions will support non-VGA based controllers with

new functions for reading and writing the palette and for access to

the flat frame buffer models.

VBE 2.0, as designed, offers the extensibility and the robustness that

was lacking in the previous specifications, while at the same time

offering backwards compatibility.

In the future, we see the need for adding supplemental specifications

for issues like Multimedia; Advanced Graphics Functions; and "Plug and

Play" features.

2.VBE Overview

This chapter outlines the various features and limitations of the VBE

standard.

2.1. VBE  Features

Standard application interface to Graphics Controllers (SVGA

Devices)

Standard method of identifying products and manufacturers.

Provision for OEM extensions through Sub-function 14h

Simple protected mode interface.

Extensible interface through supplemental specifications.

2.2. VBE Affected Devices

All frame buffer-based devices in the IBM PC platform (with the

exception of Hercules, Monochrome (MDA), CGA and EGA devices) are

suitable for use within the VBE standard to enable access to the

device by VBE-compliant applications.

2.3. Providing Vendor Information

The VGA specification does not provide a standard mechanism to

determine what graphic hardware it is running on. Only by knowing OEM-

specific features can an application determine the presence of a

particular graphics controller or display board. This often involves

reading and testing registers located at I/O addresses unique to each

OEM. By not knowing what hardware an application is running on, few,

if any, of the extended features of the underlying hardware can be

used.

The VESA BIOS Extension provides several functions to return

information about the graphics environment. These functions return

system level information as well as graphics mode specific details.

Function 00h returns general system level information, including an

OEM identification string. The function also returns a pointer to the

supported VBE and OEM modes. Function 01h may be used by the

application to obtain additional information about each supported

mode. Function 03h returns the current VBE mode.

3.VBE Mode Numbers

Standard VGA mode numbers are 7 bits wide and presently range from 00h

to 13h. OEMs have defined extended display modes in the range 14h to

7Fh. Values in the range 80h to FFh cannot be used, since VGA BIOS

Function 00h (Set video mode) interprets bit 7 as a flag to clear or

preserve display memory.

Due to the limitations of 7 bit mode numbers, the optional VBE mode

numbers are 14 bits wide. To initialize a VBE mode, the mode number is

passed in the BX register to VBE Function 02h (Set VBE mode).

The format of VBE mode numbers is as follows:

D0-D8     =    Mode number

If D8 == 0, this is not a VESA defined mode

If D8 == 1, this is a VESA defined mode

D9-D13    =    Reserved by VESA for future expansion (= 0)

D14       =    Linear/Flat Frame Buffer Select

If D14 == 0, Use VGA Frame Buffer

If D14 == 1, Use Linear/Flat Frame Buffer

D15       =    Preserve Display Memory Select

If D15 == 0, Clear display memory

If D14 == 1, Preserve display memory

Thus, VBE mode numbers begin at 100h. This mode numbering scheme

implements standard 7-bit mode numbers for OEM-defined modes.

Standard VGA modes may be initialized through VBE Function 02h (Set

VBE mode) simply by placing the mode number in BL and clearing the

upper byte (BH). 7-bit OEM-defined display modes may be initialized in

the same way.  Note that modes may only be set if the mode exists in

the Video Mode List returned in Function 0.

Note: Starting with VBE version 2.0 VESA will no longer define new

VESA mode numbers and it will not longer be mandatory to support these

old mode numbers.  However, it is highly recommended that BIOS

implementations continue to support these mode numbers for

compatibility with old software.  VBE 2.0-aware applications should

follow the guidelines in Appendix 5 - Application Programming

Considerations - for setting a desired mode.

Note:  Mode 81FFh is a special mode designed to preserve the current

memory contents and to give access to the entire video memory.  This

mode is especially useful for saving the entire video memory contents

before going into a state that could lose the contents (e.g. set this

mode to gain access to all video memory to save it before going into a

volatile power down state).  This mode is required because the entire

video memory contents are not always accessible in every mode.  It is

recommended that this mode be packed pixel in format, and a

ModeInfoBlock must be defined for it.  Look in the ModeInfoBlock to

determine if paging is required and how paging is supported if it is.

Also note that there are no implied resolutions or timings associated

with this mode.

Note:  Future display resolutions will be defined by VESA display

vendors.  The color depths will not be specified and new mode numbers

will not be assigned for these resolutions.  For example, if the VESA

display vendors define 1600x1200 as a VESA resolution, application

developers should target their display resolution for 1600x1200 rather

than choosing an arbitrary resolution like 1550x1190.  The VBE

implementation should be queried to get the available resolutions and

color depths and the application should be flexible enough to work

with this list.  Appendix 5 gives a detailed summary of the way an

application should go about selecting and setting modes.

4.VBE Functions

This chapter describes in detail each of the functions defined by the

VBE standard.  VBE functions are called using the INT 10h interrupt

vector, passing arguments in the 80X86 registers.  The INT 10h

interrupt handler first determines if a VBE function has been

requested, and if so, processes that request.  Otherwise control is

passed to the standard VGA BIOS for completion.

All VBE functions are called with the AH register set to 4Fh to

distinguish them from the standard VGA BIOS functions.  The AL

register is used to indicate which VBE function is to be performed.

For supplemental or extended functionality the BL register is used

when appropriate to indicate a specific sub-function.

Functions 00-0Fh have been reserved for Standard VBE function numbers;

Functions 10-FFh are reserved for VBE Supplemental Specifications.

In addition to the INT 10h interface, a Protected Mode Interface is

available and is described below.

4.1. VBE Return Status

The AX register is used to indicate the completion status upon return

from VBE functions.  If VBE support for the specified function is

available, the 4Fh value passed in the AH register on entry is

returned in the AL register.  If the VBE function completed

successfully, 00h is returned in the AH register.  Otherwise the AH

register is set to indicate the nature of the failure.

VBE RETURN STATUS

AL ==     4Fh: Function is supported

AL !=     4Fh: Function is not supported

AH ==     00h: Function call successful

AH ==     01h: Function call failed

AH ==     02h: Software supports this function, but the hardware

does not

AH ==     03h: Function call invalid in current video mode

Note: Applications should treat any non-zero value in the AH register

as a general failure condition as later versions of the VBE may define

additional error codes.

4.2. Protected Mode considerations

VBE services may be called directly from 32-bit Protected mode only.

For 32 bit protected mode, 2 selector/segment descriptors for 32-bit

code and the data segment are needed. These will be allocated and

initialized by the caller. The segment limit fields will be set to

64k. These selectors may either be in the GDT or LDT, but must be

valid whenever the VBE is called in protected mode. The caller must

supply a stack large enough for use by VBE and by potential interrupt

handlers. The caller's stack will be active if or when interrupts are

enabled in the VBE routine, since the VBE will not switch stacks when

interrupts are enabled, including NMI interrupts. The 32-bit VBE

interface requires a 32-bit stack.

When the VBE services are called, the current I/O permission bit map

must allow access to the I/O ports that the VBE may need to access.

This can be found in the Sub-table (Ports and Memory) returned by VBE

Function 0Ah.

To summarize, it is the calling application's responsibility to ensure

to that it has the appropriate I/O and memory privileges, a large

enough stack and appropriate selectors allocated.  It is also the

calling application's responsibility to preserve registers if it needs

them.

4.3. Function 00h - Return VBE Controller Information

This required function returns the capabilities of the display

controller, the revision level of the VBE implementation, and vendor

specific information to assist in supporting all display controllers

in the field.

The purpose of this function is to provide information to the calling

program about the general capabilities of the installed VBE software

and hardware. This function fills an information block structure at

the address specified by the caller. The VbeInfoBlock information

block size is 256 bytes for VBE 1.x, and 512 bytes for VBE 2.0.

Input:    AX   = 4F00h  Return VBE Controller Information

ES:DI   =        Pointer to buffer in which to place

VbeInfoBlock structure

(VbeSignature should be set to 'VBE2' when

function is called to indicate VBE 2.0

information is desired and the information

block is 512 bytes in size.)

Output:   AX   =         VBE Return Status

Note: All other registers are preserved.

The information block has the following structure:

VbeInfoBlock struc

VbeSignature        db   'VESA'    ; VBE Signature

VbeVersion          dw   0200h     ; VBE Version

OemStringPtr        dd   ?         ; Pointer to OEM String

Capabilities        db   4 dup (?) ; Capabilities of graphics cont.

VideoModePtr        dd   ?         ; Pointer to Video Mode List

TotalMemory         dw   ?         ; Number of 64kb memory blocks

; Added for VBE 2.0

OemSoftwareRev      dw   ?         ; VBE implementation Software revision

OemVendorNamePtr    dd   ?         ; Pointer to Vendor Name String

OemProductNamePtr   dd   ?         ; Pointer to Product Name String

OemProductRevPtr    dd   ?         ; Pointer to Product Revision String

Reserved            db   222 dup (?); Reserved for VBE implementation

; scratch area

OemData             db   256 dup (?); Data Area for OEM Strings

VbeInfoBlock ends

Note: All data in this structure is subject to change by the VBE

implementation when VBE Function 00h is called.  Therefore, it should

not be used by the application to store data of any kind.

Description of the VbeInfoBlock structure fields:

The VbeSignature field is filled with the ASCII characters 'VESA' by

the VBE implementation.  VBE 2.0 applications should preset this field

with the ASCII characters 'VBE2' to indicate to the VBE implementation

that the VBE 2.0 extended information is desired, and the VbeInfoBlock

is 512 bytes in size.  Upon return from VBE Function 00h, this field

should always be set to 'VESA' by the VBE implementation.

The VbeVersion is a BCD value which specifies what level of the VBE

standard is implemented in the software. The higher byte specifies the

major version number. The lower byte specifies the minor version

number.

Note:  The BCD value for VBE 2.0 is 0200h and the BCD value for VBE

1.2 is 0102h. In the past we have had some applications

misinterpreting these BCD values.  For example, BCD 0102h was

interpreted as 1.02, which is incorrect.

The OemStringPtr is a Real Mode far pointer to a null terminated OEM-

defined string. This string may be used to identify the graphics

controller chip or OEM product family for hardware specific display

drivers. There are no restrictions on the format of the string.  This

pointer may point into either ROM or RAM, depending on the specific

implementation.  VBE 2.0 BIOS implementations must place this string

in the OemData area within the VbeInfoBlock if 'VBE2' is preset in the

VbeSignature field on entry to Function 00h.  This makes it possible

to convert the RealMode address to an offset within the VbeInfoBlock

for Protected mode applications.

Note: The length of the OEMString is not defined, but for space

considerations, we recommend a string length of less than 256 bytes.

The Capabilities field indicates the support of specific features in

the graphics environment. The bits are defined as follows:

D0     = 0     DAC is fixed width, with 6 bits per primary color

= 1     DAC width is switchable to 8 bits per primary color

D1     = 0     Controller is VGA compatible

= 1     Controller is not VGA compatible

D2     = 0     Normal RAMDAC operation

= 1     When programming large blocks of information to the

RAMDAC use blank bit in Function 09h. i.e. RAMDAC

recommends programming during blank period only.

D3-31  = Reserved

BIOS Implementation Note: The DAC must always be restored to 6 bits

per primary as default upon a mode set.  If the DAC has been switched

to 8 bits per primary, the mode set must restore the DAC to 6 bits per

primary to ensure the application developer that he does not have to

reset it.

Application Developer's Note:  If a DAC is switchable, you can assume

that the DAC will be restored to 6 bits per primary upon a mode set.

For an application to use a DAC the application program is responsible

for setting the DAC to 8 bits per primary mode using Function 08h.

VGA compatibility is defined as supporting all standard IBM VGA modes,

fonts and I/O ports; however, VGA compatibility doesn't guarantee that

all modes which can be set are VGA compatible, or that the 8x14 font

is available.

The need for D2 = 1 "program the RAMDAC using the blank bit in

Function 09h" is for older style RAMDAC's where programming the RAM

values during display time causes a "snow-like" effect on the screen.

Newer style RAMDAC's don't have this limitation and can easily be

programmed at any time, but older RAMDAC's require the that they be

blanked so as not to display the snow while values change during

display time..  This bit informs the software that they should make

the function call with 80h rather than 00h to ensure the minimization

of the "snow-like" effect.

The VideoModePtr points to a list of  mode numbers for all  display

modes supported by the VBE implementation.  Each mode number occupies

one word (16 bits). The list of mode numbers is terminated by a -1

(0FFFFh).  The mode numbers in this list represent all of the

potentially supported modes by the display controller.  Please refer

to Chapter 3 for a description of VESA VBE mode numbers.  VBE 2.0 BIOS

implementations must place this mode list in the Reserved area in the

VbeInfoBlock or have it statically stored within the VBE

implementation if 'VBE2' is preset in the VbeSignature field on entry

to Function 00h.

Note: It is the application's responsibility to verify the actual

availability of any mode returned by this function through the Return

VBE Mode Information (VBE Function 01h) call. Some of the returned

modes may not be available due to the actual amount of memory

physically installed on the display board or due to the capabilities

of the attached monitor.

Note: If a VideoModeList is found to contain no entries (starts with

0FFFFh), it can be assumed that the VBE implementation is a "stub"

implementation where only Function 00h is supported for diagnostic or

"Plug and Play" reasons.  These stub implementations are not VBE 2.0

compliant and should only be implemented in cases where no space is

available to implement the whole VBE.

The TotalMemory field indicates the maximum amount of memory

physically installed and available to the frame buffer in 64KB units.

(e.g. 256KB = 4, 512KB = 8)  Not all video modes can address all this

memory, see the ModeInfoBlock for detailed information about the

addressible memory for a given mode.

The OemSoftwareRev field is a BCD value which specifies the OEM

revision level of the VBE software.  The higher byte specifies the

major version number. The lower byte specifies the minor version

number.  This field can be used to identify the OEM's VBE software

release.  This field is only filled in when 'VBE2' is preset in the

VbeSignature field on entry to Function 00h.

The OemVendorNamePtr is a pointer to  a null-terminated string

containing the name of the vendor who produced the display controller

board product.  (This string may be contained in the VbeInfoBlock or

the VBE implementation.)  This field is only filled in when 'VBE2' is

preset in the VbeSignature field on entry to Function 00h.  (Note:

the length of the strings OemProductRev, OemProductName and

OemVendorName (including terminators) summed, must fit within a 256

byte buffer; this is to allow for return in the OemData field if

necessary.)

The OemProductNamePtr is a pointer to  a null-terminated string

containing the product name of the display controller board.  (This

string may be contained in the VbeInfoBlock or the VBE

implementation.)  This field is only filled in when 'VBE2' is preset

in the VbeSignature field on entry to Function 00h.  (Note:  the

length of the strings OemProductRev, OemProductName and OemVendorName

(including terminators) summed, must fit within a 256 byte buffer;

this is to allow for return in the OemData field if necessary.)

The OemProductRevPtr is a pointer to a null-terminated string

containing the revision or manufacturing level of the display

controller board product.  (This string may be contained in the

VbeInfoBlock or the VBE implementation.)  This field can be used to

determine which production revision of the display controller board is

installed.  This field is only filled in when 'VBE2' is preset in the

VbeSignature field on entry to Function 00h.  (Note:  the length of

the strings OemProductRev, OemProductName and OemVendorName (including

terminators) summed, must fit within a 256 byte buffer; this is to

allow for return in the OemData field if necessary.)

The Reserved field is a space reserved for dynamically building the

VideoModeList if necessary if the VideoModeList is not statically

stored within the VBE implementation.  This field should not be used

for anything else, and may be reassigned in the future.  Application

software should not assume that information in this field is valid.

The OemData field is a 256 byte data area that is used to return OEM

information returned by VBE Function 00h when 'VBE2' is preset in the

VbeSignature field.  The OemVendorName string, OemProductName string

and OemProductRev string are copied into this area by the VBE

implementation.  This area will only be used by VBE implementations

2.0 and above when 'VBE2' is preset in the VbeSignature field.

4.4. Function 01h - Return VBE Mode Information

This required function returns extended information about a specific

VBE display mode from the mode list returned by VBE Function 00h.

This function fills the mode information block, ModeInfoBlock,

structure with technical details on the requested mode.  The

ModeInfoBlock structure is provided by the application with a fixed

size of 256 bytes.

Information can be obtained for all listed modes in the Video Mode

List returned in Function 00h.  If the requested mode cannot be used

or is unavailable, a bit will be set in the ModeAttributes field to

indicate that the mode is not supported in the current configuration.

Input:    AX   = 4F01h   Return VBE mode information

CX   =         Mode number

ES:DI   =         Pointer to ModeInfoBlock structure

Output:   AX   =         VBE Return Status

Note: All other registers are preserved.

The mode information block has the following structure:

ModeInfoBlock   struc

; Mandatory information for all VBE revisions

ModeAttributes      dw ?      ; mode attributes

WinAAttributes      db ?      ; window A attributes

WinBAttributes      db ?      ; window B attributes

WinGranularity      dw ?      ; window granularity

WinSize             dw ?      ; window size

WinASegment         dw ?      ; window A start segment

WinBSegment         dw ?      ; window B start segment

WinFuncPtr          dd ?      ; pointer to window function

BytesPerScanLine    dw ?      ; bytes per scan line

; Mandatory information for VBE 1.2 and above

XResolution         dw ?      ; horizontal resolution in pixels or chars

YResolution         dw ?      ; vertical resolution in pixels or chars

XCharSize           db ?      ; character cell width in pixels

YCharSize           db ?      ; character cell height in pixels

NumberOfPlanes      db ?      ; number of memory planes

BitsPerPixel        db ?      ; bits per pixel

NumberOfBanks       db ?      ; number of banks

MemoryModel         db ?      ; memory model type

BankSize            db ?      ; bank size in KB

NumberOfImagePages  db ?      ; number of images

Reserved            db ?      ; reserved for page function

; Direct Color fields (required for direct/6 and YUV/7 memory models)

RedMaskSize         db ?      ; size of direct color red mask in bits

RedFieldPosition    db ?      ; bit position of lsb of red mask

GreenMaskSize       db ?      ; size of direct color green mask in bits

GreenFieldPosition  db ?      ; bit position of lsb of green mask

BlueMaskSize        db ?      ; size of direct color blue mask in bits

BlueFieldPosition   db ?      ; bit position of lsb of blue mask

RsvdMaskSize        db ?      ; size of direct color reserved mask in bits

RsvdFieldPosition   db ?      ; bit position of lsb of reserved mask

DirectColorModeInfo db ?      ; direct color mode attributes

; Mandatory information for VBE 2.0 and above

PhysBasePtr         dd ?      ; physical address for flat frame buffer

OffScreenMemOffset  dd ?      ; pointer to start of off screen memory

OffScreenMemSize    dw ?      ; amount of off screen memory in 1k units

Reserved            db 206 dup (?)  ; remainder of ModeInfoBlock

ModeInfoBlock ends

The ModeAttributes field describes certain important characteristics

of the graphics mode.

The ModeAttributes field is defined as follows:

D0   =    Mode supported by hardware configuration

0 =  Mode not supported in hardware

1 =  Mode supported in hardware

D1   =    1 (Reserved)

D2   =    TTY Output functions supported by BIOS

0 =  TTY Output functions not supported by BIOS

1 =  TTY Output functions supported by BIOS

D3   =    Monochrome/color mode (see note below)

0 =  Monochrome mode

1 =   Color mode

D4   =    Mode type

0 =  Text mode

1 =  Graphics mode

D5   =    VGA compatible mode

0 =  Yes

1 =  No

D6   =    VGA compatible windowed memory mode is available

0 =  Yes

1 =  No

D7   =    Linear frame buffer mode is available

0 =  No

1 =  Yes

D8-D15 =  Reserved

Bit D0 is set to indicate that this mode can be initialized in the

present hardware configuration. This bit is reset to indicate the

unavailability of a graphics mode if it requires a certain monitor

type, more memory than is physically installed, etc.

Bit D1 was used by VBE 1.0 and 1.1 to indicate that the optional

information following the BytesPerScanLine field were present in the

data structure.  This information became mandatory with VBE version

1.2 and above, so D1 is no longer used and should be set to 1.  The

Direct Color fields are valid only if the MemoryModel field is set to

a 6 (Direct Color) or 7 (YUV).

Bit D2 indicates whether the video BIOS has support for output

functions like TTY output, scroll, etc. in this mode.  TTY support is

recommended but not required for all extended text and graphic modes.

If bit D2 is set to 1, then the INT 10h BIOS must support all of the

standard output functions listed below.

All of the following TTY functions must be supported when this bit is

set:

01   Set Cursor Size

02   Set Cursor Position

06   Scroll TTY window up or Blank Window

07   Scroll TTY window down or Blank Window

09   Write character and attribute at cursor position

0A   Write character only at cursor position

0E   Write character and advance cursor

Bit D3 is set to indicate color modes, and cleared for monochrome

modes.

Bit D4 is set to indicate graphics modes, and cleared for text modes.

Note: Monochrome modes map their CRTC address at 3B4h. Color modes map

their CRTC address at 3D4h. Monochrome modes have attributes in which

only bit 3 (video) and bit 4 (intensity) of the attribute controller

output are significant. Therefore, monochrome text modes have

attributes of off, video, high intensity, blink, etc. Monochrome

graphics modes are two plane graphics modes and have attributes of

off, video, high intensity, and blink. Extended two color modes that

have their CRTC address at 3D4h, are color modes with one bit per

pixel and one plane. The standard VGA modes, 06h and 11h would be

classified as color modes, while the standard VGA modes 07h and 0Fh

would be classified as monochrome modes.

Bit D5 is used to indicate if the mode is compatible with the VGA

hardware registers and I/O ports.  If this bit is set, then the mode

is NOT VGA compatible and no assumptions should be made about the

availability of any VGA registers.   If clear, then the standard VGA

I/O ports and frame buffer address defined in WinASegment and/or

WinBSegment can be assumed.

Bit D6 is used to indicate if the mode provides Windowing or Banking

of the frame buffer into the frame buffer memory region specified by

WinASegment and WinBSegment.  If set, then Windowing of the frame

buffer is NOT possible.  If clear, then the device is capable of

mapping the frame buffer into the segment specified in WinASegment

and/or WinBSegment.  (This bit is used in conjunction with bit D7, see

table following D7 for usage)

Bit D7 indicates the presence of a Linear Frame Buffer memory model.

If this bit is set, the display controller can be put into a flat

memory model by setting the mode (VBE Function 02h) with the Flat

Memory Model bit set. (This bit is used in conjunction with bit D6,

see following table for usage)

+-----------+-----+-----+

|           | D7  | D6  |

+-----------+-----+-----+

|  Windowed |  0  |  0  |

|   frame   |     |     |

|   buffer  |     |     |

|    only   |     |     |

+-----------+-----+-----+

|    n/a    |  0  |  1  |

+-----------+-----+-----+

|    Both   |  1  |  0  |

|  Windowed |     |     |

|    and 4  |     |     |

|  Linear   |     |     |

+-----------+-----+-----+

|   Linear  |  1  |  1  |

|   frame   |     |     |

|   buffer  |     |     |

|    only   |     |     |

+-----------+-----+-----+

The BytesPerScanLine field specifies how many full bytes are in each

logical scanline. The logical scanline could be equal to or larger

than the displayed scanline.

The WinAAttributes and WinBAttributes describe the characteristics of

the CPU windowing scheme such as whether the windows exist and are

read/writeable, as follows:

D0    =    Relocatable window(s) supported

0 =  Single non-relocatable window only

1 =  Relocatable window(s) are supported

D1    =    Window readable

0 =  Window is not readable

1 =  Window is readable

D2    =    Window writeable

0 =  Window is not writeable

1 =  Window is writeable

D3-D7 =    Reserved

Even if windowing is not supported, (bit D0 = 0 for both Window A and

Window B), then an application can assume that the display memory

buffer resides at the location specified by WinASegment and/or

WinBSegment.

WinGranularity specifies the smallest boundary, in KB, on which the

window can be placed in the frame buffer memory. The value of this

field is undefined if Bit D0 of the appropriate WinAttributes field is

not set.

WinSize specifies the size of the window in KB.

WinASegment and WinBSegment address specify the segment addresses

where the windows are located in the CPU address space.

Use D14 of the Mode Number to select the Linear Buffer on a mode set

(Function 02h).

WinFuncPtr specifies the segment:offset of the VBE memory windowing

function. The windowing function can be invoked either through VBE

Function 05h, or by calling the function directly. A direct call will

provide faster access to the hardware paging registers than using VBE

Function 05h, and is intended to be used by high performance

applications. If this field is NULL, then VBE Function 05h must be

used to set the memory window when paging is supported.  This direct

call method uses the same parameters as VBE Function 05h including AX

and for VBE 2.0 implementations will return the correct Return Status.

VBE 1.2 implementations and earlier, did not require the Return Status

information to be returned.  For more information on the direct call

method, see the notes in VBE Function 05h and the sample code in

Appendix 5.

The XResolution and YResolution specify the width and height in pixel

elements or characters for this display mode. In graphics modes, these

fields indicate the number of horizontal and vertical pixels that may

be displayed. In text modes, these fields indicate the number of

horizontal and vertical character positions.  The number of pixel

positions for text modes may be calculated by multiplying the returned

XResolution and YResolution values by the character cell width and

height indicated in the XCharSize and YCharSize fields described

below.

The XCharSize and YCharSize specify the size of the character cell in

pixels.  (This value is not zero based)  e.g. XCharSize for Mode 3

using the 9 point font will have a value of 9.

The NumberOfPlanes field specifies the number of memory planes

available to software in that mode. For standard 16-color VGA

graphics, this would be set to 4. For standard packed pixel modes, the

field would be set to 1.  For 256-color non-chain-4 modes, where you

need to do banking to address all pixels this value should be set to

the number of banks required to get to all the pixels (typically this

will be 4 or 8).

The BitsPerPixel field specifies the total number of bits allocated to

one pixel. For example, a standard VGA 4 Plane 16-color graphics mode

would have a 4 in this field and a packed pixel 256-color graphics

mode would specify 8 in this field. The number of bits per pixel per

plane can normally be derived by dividing the BitsPerPixel field by

the NumberOfPlanes field.

The MemoryModel field specifies the general type of memory

organization used in this mode. The following models have been

defined:

00h  =    Text mode

01h  =    CGA graphics

02h  =    Hercules graphics

03h  =    Planar

04h  =    Packed pixel

05h  =    Non-chain 4, 256 color

06h  =    Direct Color

07h  =    YUV

08h-0Fh =      Reserved, to be defined by VESA

10h-FFh =      To be defined by OEM

VBE Version 1.1 and earlier defined Direct Color graphics modes with

pixel formats 1:5:5:5, 8:8:8, and 8:8:8:8  as a Packed Pixel model

with 16, 24, and 32 bits per pixel, respectively. In VBE Version 1.2

and later, the Direct Color modes use the Direct Color memory model

and use the MaskSize and FieldPosition fields of the ModeInfoBlock to

describe the pixel format. BitsPerPixel is always defined to be the

total memory size of the pixel, in bits.

NumberOfBanks. This is the number of banks in which the scan lines are

grouped. The quotient from dividing the scan line number by the number

of banks is the bank that contains the scan line and the remainder is

the scan line number within the bank. For example, CGA graphics modes

have two banks and Hercules graphics mode has four banks. For modes

that don't have scanline banks (such as VGA modes 0Dh-13h), this field

should be set to 1.

The BankSize field specifies the size of a bank (group of scan lines)

in units of 1 KB. For CGA and Hercules graphics modes this is 8, as

each bank is 8192 bytes in length. For modes that don't have scanline

banks (such as VGA modes 0Dh-13h), this field should be set to 0.

The NumberOfImagePages field specifies the "total number minus one (-

1)"of complete display images that will fit into the frame buffer

memory. The application may load more than one image into the frame

buffer memory if this field is non-zero, and move the display window

within each of those pages.  This should only be used for determining

the additional display pages which are available to the application;

to determine the available off screen memory, use the

OffScreenMemOffset and OffScreenMemSize information.

Note: If the ModeInfoBlock is for an IBM Standard VGA mode and the

NumberOfImagePages field contains more pages than would be found in a

256KB implementation, the TTY support described in the ModeAttributes

must be accurate.  i.e. if the TTY functions are claimed to be

supported, they must be supported in all pages, not just the pages

normally found in the 256KB implementation.

The Reserved field has been defined to support a future VBE feature

and will always be set to one in this version.

The RedMaskSize, GreenMaskSize, BlueMaskSize, and RsvdMaskSize fields

define the size, in bits, of the red, green, and blue components of a

direct color pixel. A bit mask can be constructed from the MaskSize

fields using simple shift arithmetic. For example, the MaskSize values

for a Direct Color 5:6:5 mode would be 5, 6, 5, and 0, for the red,

green, blue, and reserved fields, respectively. Note that in the YUV

MemoryModel, the red field is used for V, the green field is used for

Y, and the blue field is used for U. The MaskSize fields should be set

to 0 in modes using a memory model that does not have pixels with

component fields.

The RedFieldPosition, GreenFieldPosition, BlueFieldPosition, and

RsvdFieldPosition fields define the bit position within the direct

color pixel or YUV pixel of the least significant bit of the

respective color component. A color value can be aligned with its

pixel field by shifting the value left by the FieldPosition. For

example, the FieldPosition values for a Direct Color 5:6:5 mode would

be 11, 5, 0, and 0, for the red, green, blue, and reserved fields,

respectively. Note that in the YUV MemoryModel, the red field is used

for V, the green field is used for Y, and the blue field is used for

U. The FieldPosition fields should be set to 0 in modes using a memory

model that does not have pixels with component fields.

The DirectColorModeInfo field describes important characteristics of

direct color modes.  Bit D0 specifies whether the color ramp of the

DAC is fixed or programmable. If the color ramp is fixed, then it can

not be changed. If the color ramp is programmable, it is assumed that

the red, green, and blue lookup tables can be loaded by using VBE

Function 09h.  Bit D1 specifies whether the bits in the Rsvd field of

the direct color pixel can be used by the application or are reserved,

and thus unusable.

D0   =    Color ramp is fixed/programmable

0 =  Color ramp is fixed

1 =  Color ramp is programmable

D1   =    Bits in Rsvd field are usable/reserved

0 =  Bits in Rsvd field are reserved

1 =  Bits in Rsvd field are usable by the application

The PhysBasePtr is a 32 bit physical address of the start of frame

buffer memory when the controller is in flat frame buffer memory mode.

If this mode is not available, then this field will be zero.

The OffScreenMemOffset is a 32 bit offset from the start of the frame

buffer memory.  Extra off-screen memory that is needed by the

controller may be located either before or after this off screen

memory, be sure to check OffScreenMemSize to determine the amount of

off-screen memory which is available to the application.

The OffScreenMemSize contains the amount of available, contiguous off-

screen memory in 1k units, which can be used by the application.

Note: Version 1.1 and later VBE will zero out all unused fields in the

Mode Information Block, always returning exactly 256 bytes. This

facilitates upward compatibility with future versions of the standard,

as any newly added fields will be designed such that values of zero

will indicate nominal defaults or non-implementation of optional

features. (For example, a field containing a bit-mask of extended

capabilities would reflect the absence of all such capabilities.)

Applications that wish to be backwards compatible to Version 1.0 VBE

should pre-initialize the 256 byte buffer before calling the Return

VBE Mode Information function.

4.5. Function 02h  - Set VBE Mode

This required function initializes the controller and sets a VBE mode.

The format of VESA VBE mode numbers is described earlier in this

document. If the mode cannot be set, the BIOS should leave the

graphics environment unchanged and return a failure error code.

Input:    AX   = 4F02h     Set VBE Mode

BX   =           Desired Mode to set

D0-D8  =    Mode number

D9-D13 =    Reserved (must be 0)

D14    = 0  Use windowed frame buffer model

= 1  Use linear/flat frame buffer model

D15    = 0  Clear display memory

= 1  Don't clear display memory

Output:   AX   =           VBE Return Status

Note: All other registers are preserved.

If the requested mode number is not available, then the call will

fail, returning AH=01h to indicate the failure to the application.

If bit D14 is set, the mode will be initialized for use with a flat

frame buffer model.  The base address of the frame buffer can be

determined from the extended mode information returned by VBE Function

01h.  If D14 is set, and a linear frame buffer model is not available

then the call will fail, returning AH=01h to the application.

If bit D15 is not set, all reported image pages, based on Function 00h

returned information NumberOfImagePages, will be cleared to 00h in

graphics mode, and 20 07 in text mode.  Memory over and above the

reported image pages will not be changed.  If bit D15 is set, then the

contents of the frame buffer after the mode change is undefined.

Note, the 1-byte mode numbers used in Function 00h of an IBM VGA

compatible BIOS use D7 to signify the same thing as D15 does in this

function.  If D7 is set for an IBM compatible mode set using this

Function (02), this mode set will fail.  VBE aware applications must

use the memory clear bit in D15.

Note: VBE BIOS 2.0 implementations should also update the BIOS Data

Area 40:87 memory clear bit so that VBE Function 03h can return this

flag. VBE BIOS 1.2 and earlier implementations ignore the memory clear

bit.

Note:  This call should not set modes not listed in the list of

supported modes.  In addition all modes (including IBM standard VGA

modes), if listed as supported, must have ModeInfoBlock structures

associated with them.  Required ModeInfoBlock values for the IBM

Standard Modes are listed in Appendix 2.

Note: If there is a failure when selecting an unsupported D14 value,

the error return should be 02h.

4.6. Function 03h - Return current VBE Mode

This required function returns the current VBE mode. The format of VBE

mode numbers is described earlier in this document.

Input:    AX   = 4F03h   Return current VBE Mode

Output:   AX   =         VBE Return Status

BX   =         Current VBE mode

D0-D13 =  Mode number

D14  = 0  Windowed frame buffer model

= 1  Linear/flat frame buffer model

D15  = 0  Memory cleared at last mode set

= 1  Memory not cleared at last mode set

Note: All other registers are preserved.

Version 1.x Note: In a standard VGA BIOS, Function 0Fh (Read current

video state) returns the current graphics mode in the AL register. In

D7 of AL, it also returns the status of the memory clear bit (D7 of

40:87). This bit is set if the mode was set without clearing memory.

In this VBE function, the memory clear bit will not be returned in BX

since the purpose of the function is to return the video mode only. If

an application wants to obtain the memory clear bit, it should call

the standard VGA BIOS Function 0Fh.

Version 2.x Note:  Unlike version 1.x VBE implementations, the memory

clear flag will be returned.  The application should NOT call the

standard VGA BIOS Function 0Fh if the mode was set with VBE Function

02h.

Note: The mode number returned must be the same mode number used in

the VBE Function 02h mode set.

Note: This function is not guaranteed to return an accurate mode value

if the mode set was not done with VBE Function 02h.

4.7. Function 04h  - Save/Restore state

This required function provides a complete mechanism to save and

restore the display controller hardware state. The functions are a

superset of the three subfunctions under the standard VGA BIOS

Function 1Ch (Save/restore state) which does not guarantee that the

extended registers of the video device are saved or restored. The

complete hardware state (except frame buffer memory) should be

saveable/restorable by setting the requested states mask (in the CX

register) to 000Fh.

Input:    AX   = 4F04h   Save/Restore state

DL   = 00h         Return save/restore state buffer size

= 01h         Save state

= 02h         Restore state

CX   =         Requested states

D0=           Save/restore controller hardware state

D1=           Save/restore BIOS data state

D2=           Save/restore DAC state

D3=           Save/restore Register state

ES:BX=         Pointer to buffer  (if DL <> 00h)

Output:   AX   =         VBE Return Status

BX   =         Number of 64-byte blocks to hold the state

buffer  (if DL=00h)

Note: All other registers are preserved.

4.8. Function 05h - Display Window Control

This required function sets or gets the position of the specified

display window or page in the frame buffer memory by adjusting the

necessary hardware paging registers.  To use this function properly,

the software should first use VBE Function 01h (Return VBE Mode

information) to determine the size, location and granularity of the

windows.

For performance reasons, it may be more efficient to call this

function directly, without incurring the INT 10h overhead.  VBE

Function 01h returns the segment:offset of this windowing function

that may be called directly for this reason.  Note that a different

entry point may be returned based upon the selected mode.  Therefore,

it is necessary to retrieve this segment:offset specifically for each

desired mode.

Input:    AX   = 4F05h   VBE Display Window Control

BH   = 00h          Set memory window

= 01h          Get memory window

BL   =         Window number

= 00h          Window A

= 01h          Window B

DX   =         Window number in video memory in window

granularity units  (Set Memory Window only)

Output:   AX   =         VBE Return Status

DX   =         Window number in window granularity units

(Get Memory Window only)

Note: In VBE 1.2 implementations, the direct far call version returns

no Return Status information to the application. Also, in the far call

version, the AX and DX registers will be destroyed. Therefore if AX

and/or DX must be preserved, the application must do so prior to

making the far call.  The application must still load the input

arguments in BH, BL, and DX (for Set Window).  In VBE 2.0

implementations, the BIOS will return the correct Return Status, and

therefore the application must assume that AX and DX will be

destroyed.

Application Developer's Note:  This function is not intended for use

in a linear frame buffer mode, if this function is requested, the

function call will fail with the VBE Completion code AH=03h.

VBE BIOS Implementation Note:  If this function is called while in a

linear frame buffer memory model, this function must fail with

completion code AH=03h.

4.9. Function 06h - Set/Get Logical Scan Line Length

This required function sets or gets the length of a logical scan line.

This allows an application to set up a logical display memory buffer

that is wider than the displayed area. VBE Function 07h (Set/Get

Display Start) then allows the application to set the starting

position that is to be displayed.

Input:    AX   = 4F06h   VBE Set/Get Logical Scan Line Length

BL   = 00h          Set Scan Line Length in Pixels

= 01h          Get Scan Line Length

= 02h          Set Scan Line Length in Bytes

= 03h          Get Maximum Scan Line Length

CX   =         If BL=00h  Desired Width in Pixels

If BL=02h  Desired Width in Bytes

(Ignored for Get Functions)

Output:   AX   =         VBE Return Status

BX   =         Bytes Per Scan Line

CX   =         Actual Pixels Per Scan Line

(truncated to nearest complete pixel)

DX   =         Maximum Number of Scan Lines

Note: The desired width in pixels or bytes may not be achievable

because of hardware considerations.  The next larger value will be

selected that will accommodate the desired number of pixels or bytes,

and the actual number of pixels will be returned in CX. BX returns a

value that when added to a pointer into display memory will point to

the next scan line. For example, in VGA mode 13h this would be 320,

but in mode 12h this would be 80.  DX returns the number of logical

scan lines based upon the new scan line length and the total memory

installed and usable in this display mode.

Note: This function is also valid in text modes.   In text modes the

application should convert the character line length to pixel line

length by getting the current character cell width through the

XCharSize field returned in ModeInfoBlock, multiplying that times the

desired number of characters per line, and passing that value in the

CX register.  In addition, this function will only work if the line

length is specified in character granularity.  i.e. in 8 dot modes

only multiples of 8 will work.  Any value which is not in character

granularity will result in a function call failure.

Note: On a failure to set scan line length by setting a CX value too

large, the function will fail with error code 02h.

Note: The value returned when BL=03h is the lesser of either the

maximum line length that the hardware can support, or the longest scan

line length that would support the number of lines in the current

video mode.

4.10.     Function 07h - Set/Get Display Start

This required function selects the pixel to be displayed in the upper

left corner of the display.  This function can be used to pan and

scroll around logical screens that are larger than the displayed

screen.  This function can also be used to rapidly switch between two

different displayed screens for double buffered animation effects.

Input:    AX   = 4F07h   VBE Set/Get Display Start Control

BH   = 00h          Reserved and must be 00h

BL   = 00h          Set Display Start

= 01h          Get Display Start

= 80h          Set Display Start during Vertical

Retrace

CX   =         First Displayed Pixel In Scan Line

(Set Display Start only)

DX   =         First Displayed Scan Line (Set Display Start

only)

Output:   AX   =         VBE Return Status

BH   =         00h Reserved and will be 0 (Get Display Start

only)

CX   =         First Displayed Pixel In Scan Line (Get Display

Start only)

DX   =         First Displayed Scan Line (Get Display

Start only)

Note:  This function is also valid in text modes.  To use this

function in text mode, the application should convert the character

coordinates to pixel coordinates by using XCharSize and YCharSize

returned in the ModeInfoBlock.  If the requested Display Start

coordinates do not allow for a full page of video memory or the

hardware does not support memory wrapping, the Function call should

fail and no changes should be made.  As a general case, if a requested

Display Start is not available, fail the Function call and make no

changes.

4.11.     Function 08h - Set/Get DAC Palette Format

This required function manipulates the operating mode or format of the

DAC palette. Some DACs are configurable to provide 6 bits, 8 bits, or

more of color definition per red, green, and blue primary colors.

The DAC palette width is assumed to be reset to the standard VGA value

of 6 bits per primary color during any mode set

Input:    AX   = 4F08h   VBE Set/Get Palette Format

BL   = 00h          Set DAC Palette Format

= 01h          Get DAC Palette Format

BH   =         Desired bits of color per primary

(Set DAC Palette Format only)

Output:   AX   =         VBE Return Status

BH   =         Current number of bits of color per primary

An application can determine if DAC switching is available by querying

Bit D0 of the Capabilities field of the VbeInfoBlock structure

returned by VBE Function 00h (Return Controller Information). The

application can then attempt to set the DAC palette width to the

desired value. If the display controller hardware is not capable of

selecting the requested palette width, then the next lower value that

the hardware is capable of will be selected. The resulting palette

width is returned.

This function will return failure code AH=03h if called in a direct

color or YUV mode.

4.12.     Function 09h - Set/Get Palette Data

This required function is very important for RAMDAC's which are larger

than a standard VGA RAMDAC.  The standard INT 10h BIOS Palette

function calls assume standard VGA ports and VGA palette widths.  This

function offers a palette interface that is independent of the VGA

assumptions.

Input:    AX   = 4F09h   VBE Load/Unload Palette Data

BL   = 00h          Set Palette Data

= 01h          Get Palette Data

= 02h          Set Secondary Palette Data

= 03h          Get Secondary Palette Data

= 80h          Set Palette Data during Vertical Retrace

with Blank Bit on

CX   =         Number of palette registers to update

DX   =         First palette register to update

ES:DI=         Table of palette values (see below for

format)

Output:   AX   =         VBE Return Status

Format of Palette Values:   Alignment byte, Red byte, Green byte, Blue

byte

Note: The need for BL= 80h is for older style RAMDAC's where

programming the RAM values during display time causes a "snow-like"

effect on the screen.  Newer style RAMDAC's don't have this limitation

and can easily be programmed at any time, but older RAMDAC's require

that they be programmed during a non-display time only to stop the

snow like effect seen when changing the DAC values.  When this is

requested the VBE implementation will program the DAC with blanking

on.  Check D2 of the Capabilities field returned by VBE Function 00h

to determine if 80h should be used instead of 00h.

Note: The need for the secondary palette is for anticipated future

palette extensions, if a secondary palette does not exist in a

implementation and these calls are made, the VBE implementation will

return error code 02h.

Note:  When in 6 bit mode, the format of the 6 bits is LSB, this is

done for speed reasons, as the application can typically shift the

data faster than the BIOS can.

Note: All application should assume the DAC is defaulted to 6 bit

mode.  The application is responsible for switching the DAC to higher

color modes using Function 08h.

Note: Query VBE Function 08h to determine the RAMDAC width before

loading a new palette.

4.13.     Function 0Ah - Return VBE Protected Mode Interface

This required function call returns a pointer to a table that contains

code for a 32-bit protected mode interface that can either be copied

into local 32 bit memory space or can be executed from ROM providing

the calling application sets all required selectors and I/O access

correctly. This function returns a pointer (in real mode space) with

offsets to the code fragments, and additionally returns an offset to a

table which contains Non-VGA Port and Memory locations which an

Application may have to have I/O access to.

Input:    AX   = 4F0Ah   VBE 2.0 Protected Mode Interface

BL   = 00h          Return protected mode table

Output:   AX   =         Status

ES   =         Real Mode Segment of Table

DI   =         Offset of Table

CX   =         Length of Table including protected mode code

(for copying purposes)

The format of the table is as follows:

ES:DI + 00h    Word Offset in table of Protected mode code for

Function 5 for Set Window Call

ES:DI + 02h    Word Offset in table of Protected mode code for

Function 7 for set Display Start

ES:DI + 04h    Word Offset in table of Protected mode code for

Function 9 for set Primary Palette data

ES:DI + 06h    Word  Offset in table of Ports and Memory Locations

that the application may need I/O privilege for.

(Optional:  if unsupported this must be 0000h)

(See Sub-table for format)

ES:DI + ?      Variable remainder of Table including Code

The format of the Sub-Table (Ports and Memory locations)

Port, Port, ... , Port, Terminate Port List with FF FF, Memory

locations (4 bytes), Length (2 bytes), Terminate Memory List with

FF FF.

Example 1. For Port/Index combination 3DE/Fh and Memory locations

DE800-DEA00h (length = 200h) the table would look like this:

DE 03 DF 03 FF FF 00 E8 0D 00 00 02 FF FF

Example 2. For only the ports it would look like:

DE 03 DF 03 FF FF FF FF

Example 3. For only the memory locations it would look like

FF FF 00 E8 0D 00 00 02 FF FF

Note:. All protected mode functions should end with a near RET (as

opposed to FAR RET) to allow the application software to CALL the code

from within the ROM.

Note:  The Port and Memory location Sub-table does not include the

Frame Buffer Memory location.  The Frame Buffer Memory location is

contained within the ModeInfoBlock returned by VBE Function 01h.

Note:  The protected mode code is assembled for a 32-bit code segment,

when copying it, the application must copy the code to a 32-bit code

segment.

Note:  It is the application's responsibility to ensure that the

selectors and segments are set up correctly.

Note:  Currently undefined registers may be destroyed with the

exception of ESI, EBP, DS and SS.

Note:  Refer to Section 4.2 for information on protected mode

considerations.

--------------------------  END OF DOCUMENT --------------------------------