DC21X4 miniport driver for Windows 95 [tm]

Version v4.31

29-Dec-1997

1. Copyright Notice:

First Edition, April 1994

Updated, December 1994

Updated, February 1996

Updated, April 1996

Digital Equipment Corporation makes no representations that

the use of its products in the manner described in this

publication will not infringe on existing or future patent

rights, nor do the descriptions contained in this

publication imply the granting of licenses to make, use, or

sell equipment or software in accordance with the

description.

Copyright Digital Equipment Corporation 1994-1997.

All rights reserved.

Do not redistribute the driver files.

Windows, Windows NT and Windows 95 are trademarks of

Microsoft Corporation.

Microsoft, MS, MS-DOS are registered trademarks of

Microsoft Corporation.

DEC, Digital Semiconductor's, Digital and the DIGITAL logo, are

trademarks of Digital Equipment Corporation.

All other trademarks and registered trademarks are the

property of their respective holders.

2. Overview:

This file explains how to install the Windows 95 driver

The driver is currently supported for motherboards and option

boards based on the Digital Semiconductor 2104x/2114x 10/100 mbps

Ethernet Controller.

3. Requirements:

A.  Hardware Requirements:

80386 or higher microprocessor based computer

with an EISA or PCI bus.

Digital Semiconductor 2104x/2114x 10/100 mbps Ethernet Controller

B.  Software Revision Requirements:

Windows 95 Final Beta Release or later

4. Files and directories:

---------------------------------------------------------------------

INSTALL.DOC         This file.

---------------------------------------------------

DC21X4.SYS          The miniport driver.

---------------------------------------------------

!DEC4250.CFG        The configuration file used by the DE425 Ethernet

board EISA configuration utility (ECU)

---------------------------------------------------

NETPCI.INF          The driver's setup file

---------------------------------------------------

NDIS.VXD            Updated Ndis wrapper

---------------------------------------------------------------------

5. Installing the Hardware Boards:

A. Installing the Digital Semiconductor's 2104x/2114x Evaluation Boards:

Refer to the Digital Semiconductor 2104x/2114x Evaluation Board

User's Guide.

B. Installing the Digital Semiconductor's 21040 EISA Adapter Card (DE425):

1. Plug the board in an EISA slot

2. Run the EISA System Configuration Utility

(ECU)

3. In the EISA System Configuration,

select the [Configure computer] option

4. Insert a floppy with the DE425 EISA configuration

file !DEC4250.CFG

5. In the main menu, select the [Save and exit] option

to complete the initialization.

The options for the DE425 in the !DEC4250.CFG file are:

1. For interrupt:

o Edge-Triggered Interrupt (default)

o Level-Triggered Interrupt (no sharing)

o Level-Triggered Interrupt (sharing)

2. Supported interrupt numbers are:

o  5

o  9

o 10

o 11

3. For cable types:

o AutoSense/AutoDetection (default)

o 10BaseT (TP)

o 10BaseT (TP) - Full Duplex

o 10BaseT (TP) - No Link Test

o 10Base2 (BNC)

o 10Base5 (AUI)

6. Installing the Driver on Windows 95:

The DC21X4 based adapters are autodetected by Windows95.

To install the driver follow the menu driven

driver's installation.

7. Parameters

To overwrite the initial values set by the EISA Configuration

Utility (ECU) or overwrite the driver's default values,

add or edit the following values in the Registry

Parameters subtree:

ConnectionType:

------------------------------------------------------------------------

Digital Semiconductor 21040 based Ethernet Controller

-------------------------------------------------------------------------

o AutoDetect                  In AutoDetect mode the driver

checks first the Twisted Pair port.

If the Link Test fails the driver

checks the BNC port. If no carrier is

detected the driver switches to the AUI

port.

o 10BaseT (TP)

o 10BaseT (TP) Full Duplex      To connect to Full Duplex hubs

o 10BaseT (TP) No_Link_Test     To connect hubs not supporting

the Link Integrity Test

o 10Base2 (BNC)                 BNC connector

o 10Base5 (AUI)                 15_pin connector

------------------------------------------------------------------------

Digital Semiconductor 21041 based Ethernet Controller

-------------------------------------------------------------------------

o AutoSense                   Senses dynamically the media ports

supported by the adapter. The precedence

is TP,BNC and AUI.

o 10BaseT (TP)

o 10BaseT (TP) Full Duplex    To connect to Full Duplex hubs

o 10BaseT (TP) No_Link_Test   To connect hubs not supporting

the Link Integrity Test

o 10Base2 (BNC)               BNC connector

o 10Base5 (AUI)               15_pin connector

------------------------------------------------------------------------

Digital Semiconductor 2114x based 10/100 mbps Ethernet Controller

-------------------------------------------------------------------------

o AutoSense                   Senses dynamically the media ports

o 10BaseT (TP)                To connect 10BaseT Ethernet hubs

o 10BaseT (TP) Full Duplex    To connect 10BaseT Full Duplex hubs

o 10Base2 (BNC)               BNC connector

o 10Base5 (AUI)               15_pin connector

o 100BaseTx (TP 100)          To connect 100BaseTx Ethernet hubs

o 100BaseTx (TP 100) FDx      To connect 100BaseTx Full Duplex hubs

o 100BaseT4                   To connect 100BaseT4 Ethernet hubs

o 100BaseFx                   To connect 100BaseFx Ethernet hubs

CacheLineSize = { 32 | 64 | 128 }

32 bytes (default)

64 bytes

128 bytes

ExtraReceiveBuffers = {value}

Number of buffers allocated by the

driver in the Receive look_aside list.

ExtraReceivePackets = {value}

Number of packets preallocated by the

driver.

InterruptMitigation = {Enabled | Disabled} (21140,21143/2)

Enable/disable the Interrupt Mitigation

algorithm.

InterruptThreshold = {number_of_interrupt/second} (default: 500)

Maximum number of interrupt/second allowed before

the Receive and Transmit interrupts get masked.

Meaningfull only when InterruptMitigation is enabled.

FrameThreshold = {number_of_frame/second} (default: 400)

Minimum number of processed frame/second allowed

before the Receive and Transmit interrupts get

unmasked.

Meaningfull only when InterruptMitigation is enabled.

EXT_MRM = { Disabled | Enabled }  (21142,21143 only)

Enable/disable PCI Memory Read Multiple command.

MRM should be enabled when the DC21X4 is connected

to the TI1130 PCI/CardBus bridge.

The default is Enabled.

EXT_MRL = { Disabled | Enabled }  (21142,21143 only)

Enable/disable PCI Memory Read Line command.

The default is Enabled.

EXT_MWI = { Disabled | Enabled }  (21142,21143 only)

Enable/disable PCI Memory Write and Invalidate command.

The default is Enabled.

Due to incompatibility problems with certain PCI host bridges

and PCI-to-PCI bridges, the driver currently does not enable

the Memory Write Invalidate (MWI) and Memory Read Line (MRL) commands

simultaneously. The driver will only enable one of the two commands;

by default, MRL will be enabled.

NwayProtocol = { Enabled | Disabled }  (21041 and

21142 Rev1.0,1.1 only)

Enable/disable the 10BT/10BT_Full Duplex Autonegotiation

medium in Autosense mode.

ReceiveBuffers = {8..64}

Number of buffers allocated by the

driver in the Receive ring.

SoftwareCRC = { 0 |  1 }  (21040 rev 1.x, 2.0 & 2.2 only)

0 The CRC is generated by the adapter

1 (default) The CRC is generated by software for

transmit packets bigger than the transmit threshold.

NOTE: This option affects performances and could be

disabled if the adapter does not report transmit

underruns in its statistic counters

UnderrunRetry =  {2,4,6,8}

Maximun number of retransmission attempts of

an underrun packet.

UnderrunThreshold = {10,20,50,100} (21140,21143/2 only)

Maximun number of underrun occurences before the

controller switches to the next threshold value.

NoCableIndication =  {Enabled | Disabled}

Check the LinkStatus before sending a packet and

retrieves NDIS_STATUS_NO_CABLE if there is not Link

indication. When it is disabled, it will retrieve the

NDIS_STATUS_SUCCESS message (workaround for the Win95

wrapper handling problem of the NO_CABLE status message).

The following values allow to overwrite the DC21X4 default setting.

Refer to the Digital Semiconductor 2104x/2114x 10/100 mbps Ethernet Controller

Engineering specifications for detailed description:

BackPressure = { 0 | 1 }  (not 21040 rev1)

CSR6<BP>    0  Disabled (default)

1  Enabled

BurstLength =  { 0 | 1 | 2 | 4 | 8 | 16 | 32 }

CSR0<PBL>   0  unlimited     (default) - (not 21040, 21041 & 21140 rev1)

1  doubleword

2  doublewords

4  doublewords

8  doublewords

16  doublewords   (default for 21040, 21041 & 21140 rev1)

32  doublewords   (not 21040 & 21041)

CaptureEffect = { 0 | 1 } (not 21040 rev1)

CSR6<CA>    0  Disabled (Default)

1  Enabled

FifoArbitration = { 0 | 1 }

CSR0<BAR>   0 Rcv/Txm Arbitration (default)

1 Round_Robin

SnoozeMode =    {Enabled|Disabled}

CFDA<SM>    0 Disabled

1 Enabled

StopBackoffCounter = { 0 | 1 }  (not 21040 rev1)

CSR6<SB>    0  Disabled (Default)

1  Enabled

StoreAndForward =  {Enabled|Disabled} (21140, 21143/2 only)

Select the StoreAndForward mode if high

PCI latencies causes 100Mbps Transmit Underruns

CSR6<SF>    1 Enabled

0 Disabled

TransmitThreshold = {value}

Transmit Threshold values for 10Mbps media.

CSR6<TTM,TR>  72 bytes

96 bytes

128 bytes

160 bytes

TransmitThreshold100 =  {value} (21140, 21143/2 only)

Transmit Threshold values for 100Mbps media.

Meaningless when StoreAndForward is set

CSR6<TTM,TR> 128 bytes

256 bytes

512 bytes

1024 bytes

8. Driver Revision number

To retrieve the driver revision number, query the Registry:

HKEY_LOCAL_MACHINE\

System\CurrentControlSet\Services\Class\Net\"num"\NDIS

MajorDriverVersion

MinorDriverVersion

9. Error messages

DC21X4_ERRMSG_REGISTRY              0x01

DC21X4_ERRMSG_ALLOC_MEMORY          0x02

DC21X4_ERRMSG_SROM                  0x03

DC21X4_ERRMSG_MEDIA                 0x04

DC21X4_ERRMSG_LOAD_CAM              0x05

DC21X4_ERRMSG_SYSTEM_ERROR          0x06

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT    0x07

9.1 Driver initialization errors:

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_REGISTRY

The AdapterType Registry's key is missing or its value is

unsupported by the adapter board

o NDIS_ERROR_CODE_ADAPTER_NOT_FOUND

No board matching the AdapterCFID Registry's key value was found

plugged into the PCI bus or in the slot specified in the Registry

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_ALLOC_MEMORY

Not enough memory to allocate space for the Adapter data block

o NDIS_ERROR_CODE_INVALID_VALUE_FROM_ADAPTER

DC21X4_ERRMSG_SROM

Invalid data read from the adapter's serial ROM

o NDIS_ERROR_CODE_NETWORK_ADDRESS

No readable burnt_in or software configured station address

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_MEDIA

The selected media port is not supported by the board.

o NDIS_ERROR_CODE_INTERRUPT_CONNECT

Interrupt initialization failed

o NDIS_ERROR_CODE_TIMEOUT

DC21X4_ERRMSG_LOAD_CAM

DC21X4 CAM initialization failed

9.2 Driver run time errors:

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_SYSTEM_ERROR

System_Error interrupt (bus parity error)

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT

Transmit Jabber timer expired

10. Hotline and Support Information:

Call the Digital Semiconductor Information Line for information and

technical support:

United States and Canada              1-800-332-2717

Outside North America                +1-508-628-4760

Ordering Digital Semiconductor Products

To order Digital Semiconductor products, contact your local

Digital Semiconductor sales office.  When working with your sales

representative, you may be able to take advantage of discounts

and volume pricing.

To order Digital Semiconductor samples or for a complete list of

products call the Digital Semiconductor Information Line.

You can order the following network products from Digital

Semiconductor:

21040-AA      21040  PCI Ethernet LAN Controller

21041-AA      21041  PCI Ethernet LAN Controller

21140-AB      21140  PCI Fast Ethernet LAN Controller

21140-AC      21140A PCI Fast Ethernet LAN Controller

21A40-01      21040 PCI Evaluation Board Kit

21A41-01      21041 PCI Evaluation Board Kit

Ordering Associated Literature

Digital Semiconductor documentation and literature is available

with the products. For a complete list and for information about

ordering, contact the Digital Semiconductor Information Line.

Ordering Third-Party Literature

You can order the following third-party literature directly from

the vendor:

Title                       Vendor

----------------------------------------------------------

PCI System Design Guide    PCI Special Interest Group

N/S HH3-15A

5200 N.E. Elam Young Pkwy

Hillsboro, Oregon 97124-6497

1-503-696-2000

----------------------------------------------------------

Electronic Orders

To place an order through your account at the Electronic Store,

dial 1-800-234-1998, using a modem set to 2400- or 9600- baud.

You must use a VT terminal or terminal emulator set at 8 bits,

no parity. If you need assistance using the Electronic Store,

call 1-800-DIGITAL (1-800-344-4825) and ask for an Electronic

Store specialist.

Telephone and Direct Mail Orders

From        Call                Write

__________________________________________________________

U.S.A.      DECdirect           Digital Equipment Corporation

Phone:              P.O. Box CS2008

1-800-DIGITAL       Nashua, NH 03061

(1-800-344-4825)

FAX: 603-884-5597

Puerto      Phone:              Digital Equipment Corporation

Rico        809-781-0505        3 Digital Plaza, 1st Street

FAX: 809-749-8377   Suite 200

Metro Office Park

San Juan, Puerto Rico 00920

Canada      Phone:              Digital Equipment of Canada Ltd.

1-800-267-6215      100 Herzberg Road

FAX: 613-592-1946   Kanata, Ontario, Canada K2K 2A6

Attn: DECdirect Sales

International  -                Local Digital subsidiary or

approved distributor

Internal    DTN 264-3030        U.S. Software Supply Business

orders      603-884-3030        Digital Equipment Corporation

FAX: 603-884-9690   10 Cotton Road

Nashua, NH 03063-1260