Release Notes - DC21X4 NDIS 3.x and 4.0 drivers

Version v4.31

29-Dec-1997

This kit includes Release v4.31 of the DC21X4 NDIS 3.x and 4.0 drivers.

Changes from previous release:

==============================

- Add support for autosensing the size of the SROM (1Kbit or 4Kbit)

and reading/writing 4Kbit SROM.

- Serialize the Link indication to the wrapper after the driver initialization.

- Set LINK_HYSTERESIS bit (CSR6<25>) for 21143 symbol Phy boards for All

mediums (10 Mbps and 100 Mbps). This will check the Link presence

for a longer period.

- The Driver now supports enabling of Extended PCI commands,

(the commands new as of PCI spec 2.1), for those 2104x/2114x

devices which support the commands.

Enabling the commands may increase PCI or CardBus throughput,

depending on the system implementation.

As a default, the driver will enable the maximum possible number of

the three Extended PCI Commands,

Memory Write Invalidate (MWI);

Memory Read Line (MRL);

Memory Read Multiple (MRM);

after verifying that the system implementation (PCI bridge chipset

and BIOS) supports it.

The devices for which the commands will be enabled:

21140A rev >= 21 ,

21143 rev >= 20.

Due to incompatibility problems with certain PCI host bridges

and PCI-to-PCI bridges, the driver currently does not enable

the Memory Write Invalidate (MWI) and Memory Read Line (MRL) commands

simultaneously. The driver will only enable one of the two commands;

by default, MRL will be enabled.

The driver has 3 new keywords EXT_MWI, EXT_MRM and EXT_MRL.

These keyword enable and disable the operation of the

respective Extended PCI Commands, to allow user override; in no

case will an invalid combination be allowed.

- New OIDs to support DMI requirements.

- Fix the "Initialize Done" delay (V4.20) for the boards containing a BNC

implementation too. Set the Link timeout to 5 seconds.

- Fix the SROM Extended Block Type 3 (Mii PHY Block) Reset sequence field parsing.

The data for every element in the sequence must be in words.

- Change the delay for the reset sequence (at init time) to 5 Miliseconds.

- Fix the CSR13-15 values while working in MII mode; the old values (wrong)

caused a performance drop while working in Autosensing mode.

- Add support for the OID: OID_GEN_MAXIMUM_SEND_PACKETS .

- Fix a potential problem where a min buffer sent from the wrapper could be

accepted for transmision while no free descriptors were available.

History:

========

Release V4.22

=============

This release will be the final release of the NT3.5 driver.

If this is problematic, please contact your Digital Semiconductor representative.

Changes from previous release:

==============================

- Support the "Surprise removal of a PC Card device", which prevents any

spontaneous reboot, system stall or blue screen if the user accidentally

pulls the PC Card out of its socket.

- Fix the link interrupt masking on 100Mbps force mode on power-up. This problem

caused the driver to fail to get the 100Mbps link.

- Fix the access to the PCI configuration registers (for PCI mechanism 1) in

fullmac driver, which can cause power-up hangs in some machines in

Windows for workgroups environments (fullmac driver only).

Release V4.20 (beta):

=====================

- Fix the masking of Link iterrupts in the autosensing algorithm,

which caused a problem in the Link indication after reconnection,

at 100Mbps speed. The problem was present in 21143/QSI based boards

and 21142/Microlinear Cardbus based boards.

- Add private OIDs for SecureON (miniport only).

- Delay the "Initialize Done" indication until the NWAY process finishes

(specially for symbol PHY's based boards), to fix a wrong speed report to SNMP.

- Add the "unattended" installation option in the INF file for Windows NT driver

(miniport).

- Set the BurstLength parameter = 16LW through the oemsetup.inf

file for the Miata machines to avoid a hang problem in those

machines for larger burst lengths.

- Modify strings for board identification and connection type.

- Add a new keyword to disable the Link check status before the

send routine for WIN95, preventing the NO_CABLE_CONNECTION

status report to the wrapper (for miniport driver only).

- Support workaround for potential hang of the Rx process in

the 21143-PB (see errata for details). This support was added

in version V4.02.

- Reset the BufferInUse variables at software reset time.

- Fix a leakage in the Free Map Registers pool (in NDIS3.0 only).

The leakage was caused by a wrong exit from the "send" routine when

the retransmision of a packet (with previous underrun) succeeded.

Release V4.16:

- Fix a no LinkStatus indication when the DC21143 card is connected through the 10Base2

port at power-up time.

Release V4.15:

- Fix link interrupt masking on 100Mbps force mode after software reset.

- Fix an Autonegotiation timing problem between 21143/QSI based adapter in one

machine and 21x4/National in the other, after software reset.

- Fix SROM V4.0 detection.

- Disable the HEARTBEAT counter in 21143/QSI based adapters.

Release V4.14:

- This release kit contains two drivers that use Microsoft's NDIS3.0

wrapper for Windows NT 4.0 and the OEM Service Release 2 (OSR2) of

Windows 95.  These drivers are located in subdirectories named

\WNT40\NDIS3.0 and \W95_osr2\NDIS3.0.  Although these drivers are fully

functional, they should not be used for Microsoft certification.  We

recommend that certification be performed using the drivers that use

Microsoft's NDIS3 wrapper.  For Windows 95, use the driver in \WIN95

or W95_osr2\NDIS31 and for Windows NT 4.0 use the driver in \WNT40\NDIS31.

- Fix a Nway problem on the old 21142/ML6692 cardubs design

- Fix Full duplex issue which caused failures on test suite

and performance drop.

- Mask interrupts on different Link statuses to improve performance.

Release V4.13:

- Final release (full NWAY support) DC21143-ML6692 board.

- Support Unlimited Burst Length option.

Release V4.12:

- Fix a DC21143-BroadcomT4 problem.

Release V4.11:

- Support Magic Packet SROM fromat.

Release V4.1:

- Final release for the 21143+QSI 6611 boards.

Release X4.1-004:

- Fixed a problem in force AUI.

- Fixed a problem in force 100-FDX with 21143+QSI

Note: The 21143+Microlinear MII Phy, works OK in the modes

10BaseT Force, 100BaseTx Force, and in NWAY mode succeeds

to do parallel detection to 10BaseT and 100BaseTx media.

The NWAY protocol doesn't work.

The 21143+QSI Phy works well in all the mediums, although

a number of LinkChange interrupts were seen in some revisions

of the QSI Phy.

Release X4.1-003:

- Add support for the DC21143 controller.

- Add support for PCI Memory_Read_Multiple command

- Fix the Adapter's deallocation resource routine

- Fix the No_Carrier and Loss_of_Carrier bogus statuses

for MII + Full_Duplex mode

- Add dynamic Txm Threshold algorithm

Release 4.03:

- Modified detection of MII phy to work on boards

where Phy Addr pins are not pulled up/down correctly.

Release 4.02:

- Includes the Overflow fix for DC21140 and DC21142

- Add the Registry key InterruptMitigation to enable

the interrupt mitigation algorithm

- Fix the oemsetup.inf file for DC21142

Release 4.01:

- First NDIS3.0 driver release

- New handling of the receive buffers and

receive packets

- Full_duplex spinlocks

- NdisIndicateStatus on link transitions

- Dynamic Line_speed and MAC_OPTIONS OIDs,

new OID_GEN_MEDIA_CONNECT_STATUS

- Add support for the Microlinear ML6692 PHY adapter.

- Add speed sensing workaround for National's

DP8340 PHY adapter.

- Modify the handling of Mii register's read failure

in the MiiPhyRegister routine.

- Add the dynamic setting of StoreAndForward transmit

mode if the number of underruns reachs the

Underrun threshold (default=10, programmable through the

Registry's key "UnderrunThreshold").

- Modify the support for Nway media autonegotiation

for the DC21041 Rev2.0 and DC21142 Rev 1.x.

- Add support for the DC21142 controller.

- Store the driver's revision information into

Windows95' Registry and WfW's protocol.ini file:

WNT's Registry:

HKEY_LOCAL_MACHINE\

Software\DigitalEquipmentCorporation\DC21X4\CurrentVersion

MajorVersion

MinorVersion

Win95's Registry:

HKEY_LOCAL_MACHINE\

System\CurrentControlSet\Services\Class\Net\"num"\NDIS

MajorDriverVersion

MinorDriverVersion

WfW's protocol.ini :

[DEC$DC21xxx]

MajorDriverVersion

MinorDriverVersion

Release 3.1:

- Single media adapter with no link indication supported

by the Autosense option

- Remove the pageable_function attribute of the 'DC21X4MediaDetect' and

'DC2114Sense100BaseTxLink' routines to allow their run_time invocations

by the Reset handler.

Release 3.0:

- Add Nway support for DC21041

- Separate transmit threshold values for 10Base and 100Base media

Both transmit threshold  values are programmable through the Registry:

- "TxmThreshold" for 10base media

- "TxmThreshold100" for 100base media (meaningless in

StoreAndForward mode)

- Fix unaligned pointers

- Add MII management support for MII's 100baseTx and

100baseT4 PHY adapters

- Updated Registry ConnectionType:

+-----+------------------+------------------------------------------+

|(dec)| Connection Type  |                                          |

+-----+------------------+------------------------------------------+

|  0  | AutoSense        |  AutoSense                               |

+-----+------------------+------------------------------------------+

|  1  | 10Base2          |  BNC,Thinwire,Coax                       |

+-----+------------------+------------------------------------------+

|  2  | 10BaseT          |  Twisted_Pair Half_Duplex                |

+-----+------------------+------------------------------------------+

|  3  | 10BaseT,FD       |  Twisted_Pair Full_Duplex                |

+-----+------------------+------------------------------------------+

|  4  | 10BaseT,NLT      |  Twisted_Pair Half_Duplex,No_Link_Test   |

+-----+------------------+------------------------------------------+

|  5  | 10Base5          |  AUI,Thickwire                           |

+-----+------------------+------------------------------------------+

|  6  | AutoSense,No_Nway|  AutoSense,Nway Disabled                 |

+-----+------------------+------------------------------------------+

|  7  |     -            |  Reserved                                |

+-----+------------------+------------------------------------------+

|  8  | 100BaseTx        |  100BaseTx(Symbol_Scrambler),Half_Duplex |

+-----+------------------+------------------------------------------+

|  9  | 100BaseTx,FD     |  100BaseTx(Symbol_Scrambler),Full_Duplex |

+-----+------------------+------------------------------------------+

| 10  | 100BaseT4        |  100BaseT4                               |

+-----+------------------+------------------------------------------+

Miniport driver only:

- Add Data copy to preallocated buffer instead of

Physical Mapping for Transmit Packets smaller or equal

to 256 bytes.

- Add Interrupt mitigation algorithm:

Interrupt/sec and Frames/sec threshold are programmable

through the Registry's keys "InterruptThreshold" & "FrameThreshold"

- Underrun retransmission. In case of underrun packets are

automaticaly retransmitted by the driver. The maximun number of

retransmission is programmable through the Registry's key

"UnderrunRetry".

Release 2.44:

- Remove BurstLength=32 option for DCc1040 & Dc21041

Release 2.43:

- Add PowerPC miniport

Release 2.42:

- Fix the PCI Mechanism 1 detection for (OPTI platforms) in the

WfW driver

- Fix the SROM parsing for 10BaseT Full Duplex mode

- Fix the DC21040's autodetect hang when the adapter is

unconnected

- Fix the DC2141's Autosense routine

Release 2.41:

- Enhance the DC21140 AutoDetection and AutoSense

routines.

- Add support for DC21140's 100BaseT4 and

100BaseFx media.

- Add support for PCI to PCI bridge in Windows for Workgroups

driver

Release 2.31:

- Number of map registers allocated by the adapter

configurable through the Registry

- Add missing NdisMCancelTimer in the Halt handler

- Add support for DE500 Fast Ethernet adapter

- Remove DE436 from the adapter list

- Add support for ZYNX 21040 boards rev. 03B and

priors

Release 2.3:

- Fix DC21040 and DC21041's Full Duplex mode

Release 2.2:

- Fix the hash filter generation

- Add optionnal Txm FIFO Threshold values

for 100Mbps in the Registry

- Miniport driver fixes for WIN95

- Add Snooze mode option for Digital Semiconductor's21041

Release 2.1:

- Modify the 10/100 link autosensing algorithm to avoid

switching media while autosensing for link pass:

With the new algorithm the media is switched only if:

- the selected media is 10 Mbps and the 100Mbps link passes:

media is switched from 10Mbps to 100Mbps.

- the selected media is 100Mbps. Its link fails but

the 10 Mbps link passes: the media is switched from

100 Mbps to 10Mbps.

- Modify the ConnectionType values in the driver

installation file to be backward compatible with the DC21040

V1.x values:

Release 2.0:

- Miniport driver for WNT

- Add support for Serial ROM format V1

- Add support for Digital Semiconductor's 21041

- Add dynamic autosense for Digital Semiconductor's 210140 & 21041

- Increase the minimal wait time for BNC

transceiver's setup to 300 milliseconds

- Fix Promiscuous filter setting

- Add support for EB140 and EB41 evaluation boards

- Poll Demand workaround for Motorola's Eagle pass1 chip set

- Support for on_board DC21040 on MR4000 platform

Release 1.2:

- Added workaround for Intel Host to PCI bridge

in WfW driver

- TransceiverDelay in 100Millisecond units

Release 1.1:

- Enhanced interrupt service routine.

- Support Digital Semiconductor's 21140 requirements for size of Multicast address table.

- Fix loopback routine

Windows NT:

- Support DE434 (PCI Ethernet Adapter) and DE436 (PCI Quad Ethernet

Adapter)

Windows for Workgroups:

- Support DE434 (PCI Ethernet Adapter).

Windows 95:

- First release

Release 1.0:

- Added support for Digital Semiconductor's 21140 Fast Ethernet Controller.

- Added static keyword to OEMSETUP.INF: AdapterCFID

- Change driver name to DC21X4.

Copyright Notice:

Digital Equipment Corporation makes no representations that the

use of its products in the manner described in this publication

will not infringe on existing or future patent rights, nor do

the descriptions contained in this publication imply the

granting of licenses to make, use, or sell equipment or software

in accordance with the description.

Copyright Digital Equipment Corporation 1994, 1995, 1996, 1997.

All rights reserved.

Do not redistribute the driver files.

Intel is a trademark of Intel Corporation.

Windows and Windows NT are trademarks of

Microsoft Corporation.

Microsoft, MS, MS-DOS are registered trademarks of

Microsoft Corporation.

Digital Semiconductor, Digital Semiconductor's, DEC, DECnet, Digital, PATHWORKS,

ThinWire, and the DIGITAL logo are trademarks of Digital

Equipment Corporation.

Magic Packet is a trademark of Advanced Micro Devices

SecureON is a trademark of Digital Equipment Corporation

All other trademarks and registered trademarks are the property

of their respective holders.

Hotline and Support Information:

Call the Digital Semiconductor Information Line for information and

technical support:

United States and Canada              1-800-332-2717

Outside North America                +1-508-628-4760

Ordering Digital Semiconductor Products

To order Digital Semiconductor products, contact your local

Digital Semiconductor sales office.  When working with your sales

representative, you may be able to take advantage of discounts

and volume pricing.

To order Digital Semiconductor samples or for a complete list of

products call the Digital Semiconductor Information Line.

You can order the following network products from Digital

Semiconductor:

21040-AA      21040  PCI Ethernet LAN Controller

21041-AA      21041  PCI Ethernet LAN Controller

21140-AB      21140  PCI Fast Ethernet LAN Controller

21140-AC      21140A PCI Fast Ethernet LAN Controller

21A40-01      21040 PCI Evaluation Board Kit

21A41-01      21041 PCI Evaluation Board Kit

Ordering Associated Literature

Digital Semiconductor documentation and literature is available

with the products. For a complete list and for information about

ordering, contact the Digital Semiconductor Information Line.

Ordering Third-Party Literature

You can order the following third-party literature directly from

the vendor:

Title                       Vendor

----------------------------------------------------------

PCI System Design Guide    PCI Special Interest Group

N/S HH3-15A

5200 N.E. Elam Young Pkwy

Hillsboro, Oregon 97124-6497

1-503-696-2000

----------------------------------------------------------

Electronic Orders

To place an order through your account at the Electronic Store,

dial 1-800-234-1998, using a modem set to 2400- or 9600- baud.

You must use a VT terminal or terminal emulator set at 8 bits,

no parity. If you need assistance using the Electronic Store,

call 1-800-DIGITAL (1-800-344-4825) and ask for an Electronic

Store specialist.

Telephone and Direct Mail Orders

From        Call                Write

__________________________________________________________

U.S.A.      DECdirect           Digital Equipment Corporation

Phone:              P.O. Box CS2008

1-800-DIGITAL       Nashua, NH 03061

(1-800-344-4825)

FAX: 603-884-5597

Puerto      Phone:              Digital Equipment Corporation

Rico        809-781-0505        3 Digital Plaza, 1st Street

FAX: 809-749-8377   Suite 200

Metro Office Park

San Juan, Puerto Rico 00920

Canada      Phone:              Digital Equipment of Canada Ltd.

1-800-267-6215      100 Herzberg Road

FAX: 613-592-1946   Kanata, Ontario, Canada K2K 2A6

Attn: DECdirect Sales

International  -                Local Digital subsidiary or

approved distributor

Internal    DTN 264-3030        U.S. Software Supply Business

orders      603-884-3030        Digital Equipment Corporation

FAX: 603-884-9690   10 Cotton Road

Nashua, NH 03063-1260