Overview:	1

Instructions For Installing PCMCIA Support	2

Frequently Asked Questions	2

Known Restrictions	4

Tips on LAN Cards (Ethernet and Token Ring)	5

Recommended Resource Assignments:	6

Tips on Fax/Modem PC Cards	6

Tips on Multi-Function PC Cards	6

Card Insertion Expectations With A Cardbus Socket	6

The STOP Button	6

BIOS Settings That Have An Affect On PC Cards	7

IRQ Issues & Constraints	7

PC card Specific Technical Tips	7

Digital DEPCM-XX LAN Cards (Ethernet)	7

Tips on Digital RoamAbout Wireless LAN Cards (Ethernet)	8

Troubleshooting	8

GENERAL TECHNICAL TIPS

# Overview:

The HiNote VP 550 - 575 features many advanced features (such as the Cirrus Logic 6832 Cardbus controller for the configuration and operation of PC cards). This document serves to provide additional technical information to help you successfully utilize all of the features built into the computer. For more general information pertaining to the set up and use of your notebook please refer to the User's Guide.

# Instructions For Installing PCMCIA Support

Digital can provide you with a driver allowing you to install and configure PC cards under the Windows 95 OPKSR1 operating system. While the addition of this driver will allow the system to support most PCMCIA cards, it will not add support for Zoomed Video or 32 bit Cardbus cards. This is a limitation of the OPKSR1 operating system not of the notebook itself.

Note: 	Before inserting any PC card into the system, first verify that you have the most up to date drivers for the peripheral. This should be accomplished by contacting the card manufacturer by phone or by Internet connection. Many times problems are all together avoided by following this simple step.

Obtain the driver “VP6832.COM” through the Internet from the Digital Equipment Home page (http://www.pc.digital.com/~ftp/index/ix_HiNote_VP500_notebook.htm), or through your Service Representative. Add the driver to the CONFIG.SYS file on the system. Typically placement of the driver should not matter, however for ease of entry you can place the driver in the last line of the file. Example of a CONFIG.SYS file follows: (Your file may look different, this example is meant to show placement of the driver only).

DOS=HIGH,UMB

FILES=50

STACKS=9,256

REM DEVICE=C:\TSY\TSYCDROM.SYS /D:TSYCD1

lastdrive=z

rem DEVICE=C:\offirq.com

Device=C:\vp6832.com

Save the file, and restart the computer. As the system restarts, press F2 to enter the System BIOS Setup Utility. In the Setup utility disable all Power Management options. Save your changes and exit to reboot the computer.

When WIN95 restarts it should now support 2 PC card slots. If you still cannot get pccard recognition upon insertion, refer to the section ‘Troubleshooting’ at the end of this document.

NOTICE:	Do not install Phoenix PCM95 onto the hard disk containing Windows 95 OPKSR1. This software will only configure on the version of Windows 95 as shipped from the factory (OPKSR2.1). The drivers required for supporting PC cards under this version of Windows are included with the operating system. If you have already installed PCM95 you should remove it through the Add/Remove Programs utility found in the Control Panel.

# Frequently Asked Questions

Question

This notebook features a Cardbus controller, will my CardBus cards be supported under Windows 95 OPKSR1?

Answer

No. Due to limitations of Windows95 OPKSR1 Cardbus and Zoomed Video PC cards cannot be supported. These PC cards are however supported under Windows 95 OPKSR2.

Question

Can I use the Power Management features built into the HiNote VP 550 - 575 while in Windows95 OPKSR1?

Answer

Absolutely, however it is good practice to disable Power Management completely if you plan on staying connected to a network through a PC card. This is also true with all SCSI devices. You may disable Power Management from the BIOS Setup utility ( F2 at boot time). Refer to the User's Guide for further instructions.

Question

What IRQ's are free and available on the HiNote VP for use with PCMCIA cards?

Answer

The available IRQ's for use with PCMCIA cards on the HiNote VP are: IRQ's 4*, 11 and 15

Note:	You can gain the use of the following IRQ's by disabling the corresponding devices in the System Setup:

IRQ 3 - Disable Serial Port (COM2)

IRQ 4 - Disable FIR/SIR Port (Infrared - COM1) *

IRQ 7 - Disable Parallel Port (LPT1)

IRQ 5 - Disable Audio

(System Setup is accessible by pressing F2 during system boot - up)

Problem - I/O Address 300h Unavailable

Several LAN PC cards may fail to install and configure correctly using their default resource settings.

Reason

Many LAN cards default to I/O range 300-33F during their installation. This address may conflict with the audio subsystem built into the HiNote VP 550 - 575.

Solution

The HiNote VP 550 - 575 requires the following I/O ranges for Audio: 220, 338 and 330. Adjust the I/O address range for the PCMCIA card to begin at I/O address 340h.

Problem - Token Ring Card Error

Starting Windows95 with a token ring network card installed results in an error.

Reason

Do to the nature of Token Ring cards, the card must be physically connected to a Token Ring Hub before booting or prior to inserting the Token Ring card into the system.

Solution

Physically attach the cable coming from the Token Ring card to a Token Ring Hub before inserting the card into one of the PCMCIA slots in the HiNote VP 550 - 575.

NOTE:

- As with all network cards, Token Ring cards should not be removed from the system once connected to the network. Once the system has shut-down you may remove the card.

- As with all Token Ring networks, the time it takes to shutdown the system may be longer than without the Token Ring card installed.

- If your Token Ring card attempts to install at I/O A20, change it to a different address (such as 340) to avoid possible conflicts with the sound devices in the HiNote notebook.

# Known Restrictions

Note:	These restrictions are subject to change. PC card manufacturers will often update their drivers to address known restrictions. Always verify that you have the latest drivers available.

- Fast Infrared support is not provided under Windows 95 OPKSR1. IrDA however is supported.

- Power Management (Suspend / Resume) is not supported on PC cards under WIN95 OPKSR1 on the VP500. Disable all power management in the system BIOS SETUP utility (F2 at boot) before using any PC cards.

- PC card hot-insertions and removals are not supported under WIN95 OPKSR1 on the HiNote VP 550 - 575. Reframe from removing any card after it has been inserted into the system. If you must remove a PC card while it is active, you should first STOP the card then remove it. Otherwise, it is best to remove the card after WIN95 has been shut down. This is particularly true in the case of LAN cards and Multifunction cards.

- Multifunction cards using IRQ 15 may experience slow performance or lose of modem connection during concurrent modem/LAN file download. Change to IRQ 11 to eliminate this anomaly.

- Ethernet and Token Ring LAN cards will lose their connection after resuming from suspend. It is recommended to disable power management whenever connected to a LAN network.

- If installed in the top slot, the Megahertz XJ10BC LAN card may cause static from the speakers during a Suspend to RAM. This will not cause any harm to your system. Digital recommends however that you disable Power Management whenever using a LAN card. This will prevent unexpected events from occurring should your system suspend while connected to a network.

- The touchpad mouse pointer may become unstable after attaching the system to a port replicator. Digital is working with Synaptics to resolve this issue. Check the Digital Home Page frequently for updated drivers.

- SCSI cards do not support Suspend & Resume. These include, but not limited to, the Adaptec Slim SCSI and the Digital SCSI II. Disable Power Management in the BIOS if you will be using a SCSI device.

# Tips on LAN Cards (Ethernet and Token Ring)

LAN cards are very sensitive to changes in environment, such as removing the card or allowing the system to suspend after attaching to a network. The best rule of operation is to boot with the network in the machine, and close down the system with the card still in the system. This will minimize your chance of losing any data do to early disconnection to your network.

# Recommended Resource Assignments:

When configuring any LAN or Multifunction PC card we recommend the assignment of the following system resources to guarantee the best results:

IRQ = 11, Memory = D400, I/O = 340

# Tips on Fax/Modem PC Cards

While most Fax/Modems can be hot-swapped or allowed to suspend, we recommend that you limit this behavior to guarantee the stability of your system. If you do hot-swap or suspend your machine, be sure to restart any application that may be using the modem card. While the modem can perform these tasks, most times the software application cannot and must be restarted. This is true in the case of HyperTerminal (included with WIN95).

# Tips on Multi-Function PC Cards

PC cards that can provide more than one major function (such as Modem and LAN) are sometimes referred to as Multifunction cards. These cards may vary in the way they function, depending on the manufacturer. Some may only require one interrupt - sharing it between it's two major functions. Others may require an interrupt for each major function. It is important to know how your particular card functions in order to ensure proper configuration on your notebook. In both cases however multi-function cards can reduce your overall power consumption by combining two devices into one. Plus they only require the use of one slot in the host system. For additional tips, review the sections for Modem and LAN cards.

# Card Insertion Expectations With A Cardbus Socket

Your new notebook computer uses the latest in hardware and software technology to identify and configure pccards. The pccard controller inside the system can handle 16 bit PCMCIA cards, 32 bit CARDBUS cards, as well as 16 bit Zoomed Video cards. When you insert (or remove) any of these cards into the system expect at least a 3 second delay before the card is configured and ready for operation. This is normal behavior.

Note:	Windows 95 OPKSR1 does not support 32 bit (CardBus) PC cards or 16 bit Zoomed Video PC cards.

# The STOP Button

It is very important that before removing any PC card from the system that you STOP the card first. To stop a PC card click on the PC card icon on the task bar one time. This will bring up a large button. Use this button to STOP the card. (Stopping times for cards varies, multi-function and LAN cards taking the longest).

# BIOS Settings That Have An Affect On PC Cards

To view or modify some of the BIOS features press F2 when prompted so during system boot-up. Once in the BIOS Configuration Utility you will find that there are several parameters that you can change that will have a direct impact on the way your computer will operate. With this in mind, be very careful of the changes you make. A few of the changeable options will also directly impact PCCARD operation. For example: if you want to use IRQ 3 for a particular pccard, but that IRQ is already being used by the serial port, you can change IRQ assignment in BIOS Setup.

- Select Peripherals

- Select Integrated Peripherals

- Select Serial Port

- Press the SpaceBar until "Disabled" shows in the selected window

This will free up IRQ 3, but will also disable the use of the serial port. You can continue to do this with any device listed in the SETUP program. But again be aware that the changes you make will have a direct impact on some part of your machine. If you want to use IRQ 5 for a particular peripheral, you will have to disable Audio completely.

# IRQ Issues & Constraints

Due to the number of built-in options in the Digital HiNote VP 550 - 575 notebook the number of remaining free IRQ's is limited. In most cases you will only be using IRQ 11 or IRQ 15 to configure a card. However, if you wish to assign a specific IRQ to a pccard you may need to disable some component(s) in the system. Refer to section in this document titled " BIOS settings that have an affect on PCCARDs" for further information.

# PC card Specific Technical Tips

# Digital DEPCM-XX LAN Cards (Ethernet)

Obtain the version 4.00 driver or later from the Digital World Wide Web site (http://www.networks.digital.com). Place it on a 3.5" diskette, and then proceed as follows:

- Boot the system to the Windows 95 desktop.

- Verify that no other LAN card drivers are loaded in the Network Components section. If there are you will have to remove them. This work around functions only if DEPCM driver is the only LAN driver installed in the system.

- Place the driver diskette into Drive A:

- Insert the Digital DEPCM-XX card into the bottom PCMCIA/Cardbus slot.

- An installation dialog window will appear prompting you to insert either a driver diskette in drive A: or the CD-ROM containing the driver. Click on "Next".

- Windows will report it was "Unable to locate a driver for this device". Ignore this message and click on "Finish".

- Click on Start-Settings-Control Panel-System-Device Manager-Network Adapters

- Double click on the line "Digital DEPCM-XX"

- Click on "Driver" tab.

- Click on "Update Drivers".

- In the "Update Device Driver Wizard" dialog box you will be asked "Do you want Windows to search for the Driver?" Choose the option "No, Select from list" then click on "Next".

- The choice "Network Adapters" will be highlighted. Click on "Next".

- Click on "Have Disk".

- In the Install from Disk" dialog window, with the defaults drive "A:\" click on "OK".

- In the "Select Device" dialog window the device "Digital EtherWORKS PCMCIA Turbo Adapter" will be selected. Click on "OK".

- You will be prompted to "Please insert the disk labeled 'Windows 95 CD-ROM', and then click on OK", click on "OK".

- The "Copying Files..." dialog appears and has the path "A:\"  showing. Hit the down arrow or up arrow keys until the path "C:\WINDOWS\OPTIONS\CABS" appears, and then click on "OK".

- When the system prompts you to specify the machine name and workgroup, click on "OK" to specify that information and other network related options. Please refer to your Network Administrator if you require assistance in setting up the network drivers and services for your particular networking environment.

- Once you have completed the setup of the network support, the system should be rebooted.

# Tips on Digital RoamAbout Wireless LAN Cards (Ethernet)

To use the DEC RoamAbout Wireless adapter on the HiNote VP550-575 (BabyBox II) you may need to change one of the files on the V 3.50 driver diskette that came with your card. The file WVLAN22.SYS in the diskette labeled Version 3.50 Drivers and Utilities for Windows 95 and Windows NT may need to be updated. This updated file allows the RoamAbout to operate on a PC with Microsoft Windows 95 SR 2.x. The latest drivers may be obtained by visiting:

http://www.networks.digital.com:80/dr/wireless/drivers.html

# Troubleshooting

# Problem - No Recognition Of PCCcards Upon Insertion

After loading the file VP6832.COM into the config.sys and restarting, pccards are still not recognized upon insertion.

# Solution

Follow the steps below to remove and reinstall PCMCIA support in the system.

- Shut down the system.

- Restart the system in SAFE MODE (press F8 at the ‘Starting Windows 95’ screen), chose #3.

- Open the Control Panel

- Open the System icon

- Click on the Device Manager tab

- Double click on the ‘PCMCIA socket’ key

- If any entry exists under this key, select it and click on Remove.

- Double click on the ‘Other devices’ key

- If any entry exists under this key, select it and click on Remove.

- Click on Okay and Shutdown the computer.

- Restart the computer and when WIN95 restarts open up the Control Panel again.

- Double click on the PCCard (PCMCIA) Icon

- Enable ‘Show Contol On Taskbar’

- Click Okay and shutdown the computer.

- When you restart you should now have full PC Card support.