# Digital’s HiNote VP550 - 575 with NT 4.0

Overview:

The intent of this document is to educate the reader on Digital’s implementation of    PCMCIA and Power Management under Windows NT 4.0 on the VP550 – VP575 notebooks.

Revisions:

- The Digital shipping release of NT 4.0 will be RC 6 (release candidate 6)

- The Microsoft version of    NT that Digital ships with this laptop is Service Pack 1 (Build 1381).    It is important to note that installation of other Service Packs is not recommended!    Digital has gone to great lengths to make sure that our shipping version of NT is dependable and reliable on our product.

- Shipping Power Profiler revision is 2.10.03

- Shipping CardWizard revision is 2.00.07

- BIOS revision must be 103A/B or higher

- Keyboard BIOS revision must be 102 or higher

Power Profiler:

Power Profiler is a product from SystemSoft that provides power management functions under Windows NT.    When NT boots, it places an icon in the task tray and places a suspend option off the start menu.    The task tray icon performs several functions.    It shows if you are plugged into AC, it shows battery charge level and it has the ability to suspend the system.    Using a right mouse click on the battery icon in the task tray performs the latter two items.    The type of suspend that is performed is determined by the suspend option in the BIOS setup.    In other words, if you have the suspend/resume button set to “Save To File” then that is how the Power Profiler icon will behave when requested to suspend.

Power Profiler works very closely with the system BIOS and has the ability to create and stop power management requests.      The two power management timers in Power Profiler are “display power down” and “hard drive power down.”    When these are enabled, all calls from the BIOS for display and hard drive are ignored.

The three power management settings for Power Profiler are:

Always:      Power Profiler display timers and hard drive timers are active and the BIOS suspend, standby, suspend and S2F should be enabled.

Never:    This means Power Profiler display and hard drive timers are disabled, as well as the BIOS display, hard drive, standby, suspend and S2F timers.

Battery:    If AC is inserted then this is the same as “never.”    If the AC is removed then this is the same as “always.”

Regardless of the Power Profiler settings, a hot key suspend request will over ride and place the system into suspend.    The exceptions to this are noted below under the “CardWizard and Power Management events” section.

Note:    Refer to your User’s Guide for details on “hot key” suspend requests.

CardWizard:

CardWizard is an excellent addition to the PCMCIA support that Windows NT provides.    The CardWizard software comes up automatically when a card is inserted into a slot and tells you if the card is configured.    If no card is inserted, then you will never see the CardWizard software.

Most cards will allow you to insert and eject while NT is running.    The exception to this is Network, SCSI and Multi-Function cards (modem + ethernet).    The result is that these cards must be inserted and removed only while the system is off.    Failure to do this will result in the system being in an unstable state.

CardWizard and Power Management events:

Network Cards – All suspend events are rejected by CardWizard when a network card is installed in a PCMCIA slot.    This includes hot key suspend requests.    If running off battery power and the level gets to a critically low state, the system will force a suspend with a network card inserted.    Under this circumstance the network card will not be functional after the system is resumed.    NT will have to be rebooted to gain all functionality back.

Modem Cards – A suspend warning message appears when a modem is inserted and a suspend is requested.    The warning can be ignored and the system will suspend.    Upon resuming, the card will be re initialized and will work properly.    It is however important to note that any modem applications should be closed before a suspend is allowed.

PCMCIA Hard Disks (ATA cards) – All power management events are allowed to occur while an ATA card is inserted as long as no files are open on the card.    If files are open, all suspend requests are rejected including hot key requests.

SCSI Cards –    A suspend warning message appears when a SCSI card is inserted and a suspend is requested.    The warning can be ignored and the system will suspend.    Upon resuming, the SCSI card will NOT be reinitialized but you can continue to work on the system.    This same limitation holds true on low battery suspend.

Resource assignments for network cards:

Most network cards default to an IRQ and memory range that can’t be used in the VP550 – 575 system.    When installing a network card, the preferred settings are IRQ 11 or 15, MEM of D000 and I/O of    340 (IRQ 5 and I/O 320 are used by the built in sound card).

How to get a DEC RoamAbout to work:

When the RoamAbout card is installed and NT reads the setup information from the install diskette, the card will not be configured properly.    This is because the oemsetup.inf default memory address is D800.    Unfortunately, the network properties window doesn’t show this resource and doesn’t allow it to be changed.    The way to get the card to work is to manually edit the oemsetup.inf on the install diskette for the RoamAbout.      Here is the excerpt from the .inf file that will need to be changed:

DPPCCARDAttributeMemoryAddress = 884736          ;; Default MEM D8000

;;;;        DPPCCARDAttributeMemoryAddress = 851968          ;; Alternate MEM D0000

All you will need to do is place a semicolon in front of the default memory address (at the beginning of the line) and remove the semicolons from the alternate memory address (also at the beginning of the line).    Now the card can be installed and it will work properly.    If the card was already installed then it would have to be removed and then added again through the network icon in the control panel.

Installing the Megahertz EM1144-T:

NT 4 doesn’t have a driver for this card and Megahertz has not yet developed one either.    Digital has found that the XJEM2288 driver works just fine with this card.    The XJEM2288 driver is native to NT 4 and therefore can be selected off it’s default list of network cards in the Megahertz section.

Webex software installation:

When installing the Webex software, it is important to install NT’s RAS support.    Failure to do so will result in an error message regarding RASAPI32.DLL when you try to launch Webex.    All you need to do is go back and install RAS support by double clicking on “Dial up Networking” in “My Computer” and the application will work properly.

Viewing PCMCIA Hard Disks and Flash cards in “My Computer”:

NT doesn’t dynamically update the “My Computer” screen when a PCMCIA Hard Disk or Flash card is inserted.    “My Computer” can be closed and reopened or you can refresh the screen to see the new drive letter.    To refresh the screen press F5 or click “View” and then “Refresh.”

Additional Information:

Power Profiler and CardWizard have online help that can be accessed through the “help” button in their respective program.    In addition, there is a readme.txt located in the c:\program files\systemsoft\cardwizard directory.

Upgrading CardWizard to a newer version:

Note:    The first step to upgrading CardWizard is to obtain the newest version from Digital’s Web site (www.pc.digital.com).    As of the writing of this document, 2.00.09 is the newest revision and below is the procedure for updating to it.

1. Insert your floppy with Card Wizard 2.00.09 into Drive A:

2. Click on Start button on Task bar

3. Click on RUN

4. Type    A:\setup.exe    ,    Click OK

5. Click NEXT in Card Wizard Setup window

6. Click NEXT in Welcome window

7. Make sure all PC cards are removed and click YES to continue.

(However, if you have a LAN card installed you will

have to shut down the machine to remove it, then start

procedure from the beginning.)

8. You will see a message stating that setup has detected a

previous version of Card Wizard. You must uninstall the

previous version before you can install the upgraded

version. Click YES to uninstall previous version.

9. Remove floppy and click OK to Restart computer. ( You must

restart before re-installing)

10. Log in normally. During rebooting process, you will see a

message that "at least one service or driver has failed".

This occurs because Card Wizard is no longer installed.

After installing the new version you won't get this message.

11. Click Start button, and click RUN.

12. Re-insert floppy and type A:\setup.exe

13. Click NEXT in first two windows

14. Click YES in Question window

15. Click NEXT in the Choose Destination window

16. Uncheck "SSINTEL" and check "SSCL6832" and click NEXT

17. Click FINISH to restart your computer. Remove floppy.

18. Installation complete. You can verify Card Wizard version

by running Card Wizard, select Help\About Card Wizard

from menu bar.