VP700 SERIES      SYSTEM BIOS UPGRADE PROCEDURE

----------------------------------------------------------------------------

Version : 33H0108ab.ROM and 33H1105a.ROM        25-June-1998

PLEASE NOTE :

1. The system must be on A/C power to upgrade the BIOS properly.

2. Updating the System BIOS will reset all of your BIOS settings to FACTORY DEFAULT.

3. Remember to make any custom settings (IrDa enabled, Passwords, etc.) to the System BIOS when the PHLASH procedure is complete.

4. If you fail to set your BIOS settings to Factory Default before PHLASH of the BIOS, you may run the procedure again with no problems.

Preliminary steps :

The following files must be put on a bootable floppy:

Platform.bin (to uniquely match BIOS and system together)

Phlash.exe      (to 'Phlash' the new BIOS on the system)

33H0108A.ROM (Without H/W WaveTable) (VP703,710,714,715,717,725)

33H0108B.ROM (With H/W WaveTable) (VP735,745)

Release.txt    (For VP700-745)

33H1105A.ROM (VP765 only)

Rel_note.txt (for VP765 only)

BIOS_instructions.txt (This File)

You may either create a bootable floppy first by using    'Format A: /S' and then copy the BIOS Update files to the floppy    diskette, or place the files on a    floppy and use the MS-DOS Prompt and type 'SYS A:'. This will place the system files from your    system on the floppy diskette, making it bootable.)

Phlash Procedure :

1. Turn the system OFF. You should use the Operating System    Shutdown procedure for this step (Start Button, Shutdown, select Shutdown.    Do not select    Restart). You may also force a shutdown by holding down the Fn key and pressing the Power/Suspend button until a beep is heard (about 1 second), release the power button first and the system should shut off, then release the Fn key.    This last method may cause some Operating Systems, other than DOS, to shut down improperly. Be sure that you use the Operating System Shutdown procedure when possible.

2. Insert the floppy and press the Power/Resume button to cold boot the system.

3. When the system is booting, press F2 to enter BIOS setup.

4. Use the Arrow keys to enter the SAVE menu, select GET DEFAULT VALUES and press <ENTER>

5. Press <ENTER> again to accept SAVE SETTINGS AND EXIT.

6. Type (VP700-745) PHLASH 33H0108x.ROM<ENTER>

(x=A for No WaveTable and x=B for WaveTable)

or (VP765)          PHLASH 33H1105a.ROM<ENTER>

7. An information screen will display the progress of    the procedure. (Reading the floppy file, checking ROM memory and writing the file, etc.)

8. When complete, a screen will instruct you to press a key to reboot, remove the floppy diskette and press any key and the system will reboot

9. The system may ask you to press F2 to continue; if so, press F2 and then <ENTER> to accept the change to FACTORY DEFAULT settings.

10. Press right arrow to move to Exit and Save Settings and press <ENTER>. The system will reboot.

A second PHLASH procedure is recommended to insure that all

settings have been updated properly. See repeated steps below.

11. Type (VP700-745) PHLASH 33H0108x.ROM<ENTER>

(x=A for No WaveTable and x=B for WaveTable)

or (VP765)          Phlash 33H1105a.ROM<ENTER>

12. An information screen will display the progress of    the procedure. (Reading the floppy file, checking ROM memory and writing the file, etc.)

13. When complete, a screen will instruct you to press a key to reboot, remove the floppy diskette and press any key and the system will reboot

14. The system may ask you to press F2 to continue, press F2 and then <ENTER> to accept the change to FACTORY DEFAULT settings.

15. Press right arrow to move to Exit and Save Settings and press <ENTER>. The system will reboot.

Second PHLASH of system is complete.

16. When the system is booting, press F2 to enter BIOS setup and

make your CUSTOM changes to the System BIOS.

17. Procedure to PHLASH the BIOS and reselect your default

settings is complete.