CMD Technology

INSTALL.DOC

March 21, 1995

The purpose of INSTALL.INI file is to provide user configuration of the

install program.  The INSTALL.INI file has several sections, each headed

by a word enclosed in brackets.  The following is the list of sections in

this file:

[CSA64XX]

[DosDriver Options]

[DestinationDirs]

[SourceDiskFiles]

[SourceDisksNames]

The order of the sections is not important.  Each section is followed by a

number of lines.  The syntax and meaning of each line is defined in the

subsequent sections.

[CSA64XX] Section

-----------------

This section gives you an option to install different drivers for different

CSA-64XX controller card in a system.  Each line in this section begins with

a keyword followed by an equal sign.  Each keyword controls different ways

the install program installs files on your system.  On the right hand side of

each equal sign are values assigned to the keyword.  The order of these lines

is not important.  The following is the list of all the keywords:

DosDriver

WindowsDriver

NetwareDriver

DosDriver

---------

This option installs DOS driver or skips DOS driver installation.  It

has a value of YES, NO or a question mark.  If this keyword is not present

or if it is commented out with a semi-colon in front of the line, then

DOS driver is installed.  If the value is a question mark, then the user

is prompted before installing the DOS driver.

examples:

;install DOS driver

DosDriver=YES

or

;prompt the user for DOS driver installation

DosDriver=?

or

;skip DOS driver installation

DosDriver=NO

or

;install DOS driver

;DosDriver=NO

WindowsDriver

-------------

This option installs Windows driver or skips Windows driver installation.

It has a value of YES, NO or a question mark.  If this keyword is not

present or if it is commented out with a semi-colon in front of the line,

then Windows driver is installed.  If the value is a question mark, then

the user is prompted before installing the Windows driver.

examples:

;install Windows Driver

WindowsDriver=YES

or

;prompt the user for Windows driver installation

WindowsDriver=?

or

;skip Windows driver installation

WindowsDriver=NO

or

;install Windows driver

;WindowsDriver=NO

NetwareDriver

-------------

This option installs Netware driver or skips Netware driver installation.

It has a value of YES, NO or a question mark.  If this keyword is not

present or if it is commented out with a semi-colon in front of the line,

then Netware driver is installed.  If the value is a question mark, then

the user is prompted before installing the Netware driver.

examples:

;Install Netware Driver

NetwareDriver=YES

or

;prompt the user for Netware driver installation

NetwareDriver=?

or

;Skip Netware Driver Installation

NetwareDriver=NO

or

;Install Netware Driver Installation

;NetwareDriver=NO

[DosDriver OPTIONS] Section

---------------------------

This section allows users to set command line parameters for the DOS driver.

Each line in this section begins with a keyword followed by an equal sign.

Each keyword adds one command line parameter to the driver.  On the right

side of each equal sign are values assigned to the keyword.  The order

of these lines is not important.  The following is the list of all the

keywords:

LBA Mode

Advanced Timing

Local Bus Speed

Secondary Drive

Secondary Irq

Other Options

If any of the above keywords or a whole section of a keyword is missing then

the corresponding parameter(s) will not be set.  If a semicolon ";" is placed

in front of a line then that line is ignored.  If the value on the right

side is a question mark "?" or blank, the user will be prompted for the value

of that option during installation.  The following is the explanation of all

the keywords used in this section:

Advanced Timing

---------------

This option sets the IDE drive timings.  The value on the right hand side

of the equal sign can be a YES or NO or a question mark.  If the value is

YES then the Local Bus speed is obtained from the option "Local Bus Speed"

defined in the next paragraph.  If  the value is NO, it will skip the

"Local Bus Speed" option even though it is present.  If it is a question

mark, then the user is prompted for this option during installation.  If

this keyword is not present at all, then it will not set any IDE drive timings.

example:

Advanced Timing=YES

or

Advanced Timing=NO

or

Advanced Timing=?

Local Bus Speed

---------------

If the "Advanced Timing" option is selected, the local bus speed value will

be expected.  The user can type a value on the right hand side of the equal

sign (Mhz) or a question mark if for interactive prompt.  This option is

valid only if user selects YES for Advanced Timing option.

example:

Local Bus Speed = 33Mhz

or

Local Bus Speed = 50Mhz

or

Local Bus Speed = ?

Secondary Drive

--------------

If the user has a drive connected to their secondary IDE port, the install

program can be able to scan the drive(s) in that port. The user can put a

YES or NO for this option.  If the value is YES, then the install program

starts scanning the secondary IDE port without prompting the user

before scanning.  The install program skips scanning the secondary

channel if the value is NO.  If it is a question mark, then the user is

prompted for this option during installation.

example:

Secondary Drive=Yes

or

Secondary Drive=NO

or

Secondary Drive=?

or

;Secondary Drive=

Secondary Irq

------------

This option is valid only if "Secondary Drive" option is selected.  The

right hand side of the equal sign is the hardware interrupt number for the

secondary channel.

example:

Secondary Irq=14

or

Secondary Irq=?

LBA Mode

--------

This options stands for Logical Block Addressing mode.  It can be set to

ON or OFF.  If ON is selected then the install program sets the Logical

Block Addressing Mode.  CHS mode is used if the value is OFF.

example:

LBA Mode=ON

or

LBA Mode=OFF

or

LBA Mode=?

[DestinationDirs] Section

-------------------------

This section lists the destination directories for coping driver files.

Each line in this section begins with a keyword followed by an equal sign.

On the right hand side of each equal sign is the path of the destination

directory.  The order of these lines is not important.  The following is

the list of all the keywords:

DosDriverFiles

Windows/WFWG 3.1x

NetwareDriverFiles

DosDriverFiles

--------------

The install program copies all the DOS driver(s) and utility files to

a directory on the user's hard disk.  The user can specify the path

on the right hand side of the equal sign or type a question mark.

If the path is specified in this option, then the user is not prompted

during installation.  If a question mark is detected, the user will be

prompted for the path.  If this keyword is not present at all, then

the default directory "C:\QUICKCMD" will be created and the files will be

copied to that directory.

example:

DosDriverFiles=c:\IDE

or

DosDriverFiles=30\IDE

or

DosDriverFiles=?

Windows/WFWG 3.1x

-----------------

The install program installs the FastDisk driver only if the user

has Windows/WFWG 3.1x installed on their system. The value on the right

hand side of the equal sign must be the user's Windows/WFWG 3.1x directory

or it can be a question mark.  If it is a question mark, the user is prompted

for the directory to install the driver.  If this keyword is not present,

it is assumed that the user does not have Windows/WFWG 3.1x installed in the

system.

example:

Windows/WFWG 3.1x=c:\windows

or

Windows/WFWG 3.1x=?

or

;Windows/WFWG 3.1x=

NetwareDriverFiles

------------------

The install program installs the netware driver if the system will be

used as a Novell Netware 3.1x server.  The value on the right hand side of

the equal sign must be the directory where the Netware Drivers will be

installed.  If it is a question mark, then the user is prompted for this

options during installation.  If this keyword is not present at all, then

it is assumed that the user will not use the system for Novell Netware 3.1x

server.

example:

NetwareDriverFiles=c:\netware

or

;NetwareDriverFiles=c:\netware

SourceDisksFiles Section

-----------------------

This sections lists all the source files required for the installation.

All the entries in this section have the following syntax:

SourceFileName,DiskNumber,SourceFilePath

example:

CMD640X.VL,1,DOS

CMD640X.PCI,1,DOS\PCI

DOSUTILS.EXE,1,DOS

SourceDisksNames Section

------------------------

This section provides information required by the "SourceDisksFiles" section.

It contains the information regarding each of the disks that are used in the

installation.

DiskNumber="Disk Description", DiskLabel

example:

1="CMD 640x Install Disk"," Disk#1"