# Tech Tip

# Date: 07/18/97

External Monitor / Projection Systems Display Problem

in Digital VP 500 Series Notebook

Product:    		DIGITAL HiNote VP500 notebooks

Model:    		HiNote VP550, VP560, VP565, VP575

Part # Affected:    	HiNote VP500 family    	FR-PDxxx-xx, FR-PExxx-xx

Affected Period:    	N/A

OS Type and Version:    WIN 95    OS 2.1

ECO/FCO Issued:    	N/A

Summary:

With BIOS 1.03 and below, if an external CRT or Projection System is attached to a VP550 – VP575 it is possible that for certain CRT’s or Projection Systems    there will be no external display.

Description:

The BIOS for VP550 – VP575 systems was developed with an auto detect feature for external CRT’s attached.    If an external CRT is not attached to the laptop and the FN+F4 keys are pressed, the system would sense that an external CRT was not attached and no attempt would be made to display to the external CRT.    Conversely, if an external CRT is attached to the laptop and the FN+F4 keys are pressed, the system would sense that an external CRT was attached, and display to the external CRT.    With BIOS 1.03 and below this feature was not functioning properly.    Some external CRT’s or Projection systems would be detected and others would not.

Solution:

With BIOS 1.06 this problem has been resolved by disabling the auto detect feature, those external CRT’s that did not display, will display when attached and the FN+F4 keys are pressed.    We have found that Projection Systems connected to the VGA port also work with this BIOS.    BIOS 1.06 can be downloaded from the External Web site, having the file name BGHNV55C.EXE.