Release Notes

for

Graphics Support Services Software Version 4.6 for

PowerStorm 4D40T/4D50T/4D51T/4D60T

on

Microsoft Windows NT Version 4.0

For Digital Alpha Systems

March 16th, 1998

This document contains important information about Graphics Support Services Software

Version 4.6 for the PowerStorm 4D40T, 4D50T, 4D51T and 4D60T 3D graphics accelerators.

Digital Equipment Corporation

Maynard, Massachusetts

Digital Equipment Corporation makes no representations that the use of its products in the manner described in this publication will not infringe on existing or future patent rights, nor do the descriptions contained in this publication imply the granting of licenses to make, use, or sell equipment or software in accordance with the description.

Possession, use, or copying of the software described in this publication is authorized only pursuant to a valid written license from Digital or an authorized sublicensor.

© Digital Equipment Corporation 1998. All rights reserved.

The following are trademarks of Digital Equipment Corporation:

Digital, PowerStorm and the DIGITAL logo.

Microsoft is a registered trademark, and Windows NT is a trademark of Microsoft Corporation.

OpenGL is a trademark of Silicon Graphics, Inc.

All other trademarks and registered trademarks are the property of their respective holders.

Table Of Contents

Obtaining Kit Updates

Features/System Requirements

Known Problems in this Release

Known Restrictions In This Release

1

Obtaining Kit Updates

From time to time, additional Graphics Support Services Software kits may be made available on the Internet. Please check before entering a problem report to see that you have the most recent version of this software.

Kits on the Internet

Kits may be obtained from the Internet by following these steps:

1.	Using a web browser, connect to the site:   http://www.service.digital.com/open3d/

2.	Read the legal document (choose Legal) before proceeding further.

3.	You can retrieve the Graphics Support Services Software installation guide (in PostScript format) from this site.

4.	The Release notes are also directly readable from the web page, in Hyper text (.html) format. It is advisable to retrieve and read this document before obtaining the kit.

5.	Retrieve the kit. It is in .zip format

6.	On your local machine, unzip the kit as follows:

> unzip <filename>.zip

2

Features/System Requirements

System Requirements

The system requirements are:

- Microsoft Windows NT Version 4.0 for Digital Alpha with Service Pack 3 (SP3)

- PowerStorm 4D40T, 4D50T, 4D51T or 4D60T graphics cards

- 64MB of memory

- Hardware Features

- Graphics Support Services Software takes advantage of the following hardware features:

- Graphics engine

- Texture processor

- One or two dual resolvers (rasterizers)

- An IBM RGB640 RAMDAC

- 16 MB or 32 MB of video RAM

- Built in cursor device

- Clock generation circuits

- Optional texture storage of 1, 4, or 8 megatexels (4, 16, or 32 MB)

- Support of Interlace and Frame Sequential Stereo output

- Other features supported by this hardware include:

- Line drawing

- Stippled polygon filling

- Double buffering

- Alpha buffering/blending

- Z buffering

- Antialiased lines

- New Features this release

- There are no additional features beyond the V4.4/V4.5 release of this software. The V4.6 release contains only problem fixes.

- Problems fixed in this release are:

- Performance has been increased for most textured applications. In previous versions (V4.5 and earlier) a suboptimal algorithm was used for loading and allocating texture memory. This has been greatly improved. Example applications which use 2 or more textures have experienced performance increases from 5% to 25%.

- In previous releases certain GL evaluators could be forced to floating point exceptions. This could cause an application to exit under some circumstances. This has been corrected.

3

Known Problems in this Release

The known problems for this release of Graphics Support Services Software are listed in the following sections.

Fog and textures with alpha values

If you have a texture which contains alpha values and try to fog it will not properly fog. This gives the effect of the unfogged object standing out in a fogged environment. We are investigating this, watch our web site (http://www.service.digital.com/open3d/ ) for beta releases which may have fixes for this problem.

Problems with Texture and initial primitives

In some cases the when a texture was applied to a triangle strip or similar primitive the first triangle will have invalid values for texture parameters. This causes the texture to be incorrectly displayed.  For the present a workaround is available setting IG_USESOLPATH to 0 (set IG_USESOLPATH=0). For a future release an actual fix will be made available please watch our web site (http://www.service.digital.com/open3d/ ) for beta releases which may have fixes for this problem.

Conflicts between PowerStorm 4D40T/50T/51T/60T driver and PowerStorm 3D30/4D20 cards

If  you have a system which has the drivers for the PowerStorm 4D40T/50T/51T/60T and you remove the PowerStorm 4D40T/50T/51T/60T and replace it with a PowerStorm 3D30/4D20 you will experience a system crash on the Digital Personal Workstation (433a,500a,600a) series. To avoid this always install the drivers for the PowerStorm 3D30/4D20 and then install the card on a Digital Personal workstation series system. This makes sure that the drivers for the PowerStorm 4D40T/50T/51T/60T are correctly disabled and will NOT try to run with the PowerStorm 3D30/4D20 in place.

PowerStorm 4D51T and V4.3 and earlier drivers

Drivers prior to V4.4 do not correctly support the 4D51T. They will experience a system crash if the 4D51T is inserted before the new version of the driver is installed. We thus strongly recommend that in a system being upgraded to a 4D51T the driver be upgraded first and then the board.

Problem with V4.1 and an intervening board

The system can be made to crash if the following steps are performed

1.V4.1 or 4.0 of the PowerStorm 4D40T/50T/51T/60T driver is present

2.A different card and its driver is installed

3.A kit for the 4D40T/50T/51T/60T version 4.2 or later is installed with  card from step 2 present

4.The PowerStorm 4D40T/50T/51T/60T  is inserted

In this case the system will crash for cards other than a PowerStorm 4D51T. It is recommended that installations of the PowerStorm 4D40T/50T/51T/60T driver always be performed with the card present, and the previous version of the driver present when possible. If you have performed steps 1-4 and the system will not boot perform the following steps

1.Put in a board other than a 4D40T/50T/51T/60T (step 2 above assumes you have one or you could not be in this state.

2.cd to %windir%\system32\drivers. Delete the file pbxgivdd.sys (the miniport for V4.1 and earlier)

3.Insert the PowerStorm 4D40T/50T/51T/60T and reboot.  You should come up in either VGA or with the driver working (what happens depends on the success of the previous PowerStorm 4D40T/50T/51T/60T install).

4.If the driver is operating you are done, if you are in VGA install the V4.6 driver as per usual.

Warning when PowerStorm 4D40T/50T/51T/60T driver present, but no card is present

When the PowerStorm 4D40T/50T/51T/60T is replaced by another card it leaves behind its control panel tab application. This happens because the PowerStorm 4D40T/50T/51T/60T driver does not know it has been replaced, and the new driver has no knowledge of the file which needs to be removed. The warning

"Unable to open file PBXGI.DAT, it may be missing or corrupted" appears in a pop up. The warning is harmless though annoying. If you do not wish to see the warning simply rename or delete the file %windir%\system32\pbxgivca.cpl. Please not that if you put the PowerStorm 4D40T/50T/51T/60T back in the machine without putting the file pbxgivca.cpl back you will be unable to affect the settings of the PowerStorm 4D40T/50T/51T/60T driver. In that case reinstalling the driver would restore the system to its normal state. We will investigate improving this behavior in future releases.

GIQ setting not preserved in resolution changes

In changing resolutions without a reboot the GIQ setting does not seem to be propagating correctly. In particular if you are at one resolution and have GIQ enabled and switch resolutions without a reboot the GIQ setting may be lost. We recommend that for the present, if GIQ is desired it always be explicitly set after a resolution change.

Degenerate Primitives may produce output

Degenerate primitives (such as a quad with only 3 vertices) will sometimes produce visible objects. OpenGL specifies that degenerate primitives should produce no output. With the environment variable IG_usesolpath set to 0 this problem will not occur, though at a severe performance penalty.

4

Known Restrictions In This Release

The following restrictions are known to be in this release of Graphics Support Services Software Version 4.6 for Microsoft Windows NT™ for PowerStorm 4D40T/50T/51T/60T devices.

OpenGL Windows must be Repaired using OpenGL

This is not so much a restriction as the nature of a board that supports per window buffer selection. When an OpenGL application experiences a WM_PAINT event that is over an OpenGL window, it is not valid to fix the damaged window with a Win32 call. In addition, there is no guarantee that the back buffer will be unaffected by an exposure event. This is because all Win32 writes occur to only one of the hardware buffers (buff0).

At any time an OpenGL application may be displaying either buff0 or buff1 (and multiple GL windows may have different buffers displayed). This differs from most 3D cards, where an OpenGL buffer swap implies a full screen swap.

If buff1 is displayed, GDI writes will propagate to the hidden buffer (buff0). Future swaps will bring forward the (now incorrect) buff0. To avoid this, either make sure pop-ups and other windows you create are controlled by the windowing system, so no paint event is generated. Alternatively, perform your saves and repairs with OpenGL and make sure to repair both the front and back buffers.

MIPMAPs may not Reproduce as Expected

Mipmaps may be incorrectly blended. The program MIPMAP.EXE from the OpenGL red book demonstrates this problem clearly.

Back-Face Culling

Back-face culling may be incorrect when the transformation produces a mirror image.

Mipmaps and Large W Values

If the range of 1/W (inverse of W) values for vertices is very large, then texture-mapping with MIPMAPs may exhibit artifacts.

Multihead Configurations not Supported

Although the video configuration application provides support for a limited number of multihead configurations, these configurations are not supported in this version.

Complex Cursors Reduce OpenGL Performance

Cursors that use more than a foreground and background color (for example, the 3D-Bronze cursor set or many of the animated cursors) will cause frame rates of 3D applications to slow appreciably to ensure coordination between the cursor and the window.

Polygon Antialiasing not Supported

There is no hardware support for antialiased polygons. Full-scene antialiasing must be performed via other multi-pass techniques such as a random jitter using an alpha buffer. The antipoly program in the OpenGL red book demonstrates this clearly.

Wide Points not Supported

All OpenGL points will be drawn with a size of 1.0, as sized points are not supported.

Device Independent Bitmap Performance

OpenGL rendering to a device independent bitmap is not accelerated by the PowerStorm hardware. Drawing to this type of surface will not yield performance consistent with the performance of the PowerStorm 4DT series hardware.

Stretch Blitting is not supported in hardware.

The PowerStorm 4DT series provides no hardware acceleration for the Stretch Blit operation. Performance of this operation is limited by the host software.

Some Icons in wrong colors

Some of the "icons" used may appear incorrect for colors (for example the MSDOS Icon for a command window). This is due to the fact that the 4DT series represents its pixels with red as the most significant byte (RGB) and Windows sometimes expects 24 bit information to be blue as most significant byte (BGR). We are investigating fixing this via a user settable option like GIQ compliance in a future release. This will NOT be the default because of the severe performance penalty paid to perform the byte swap.

Environment Variables for Diagnosing Problems

Two environment variables are available to aid in diagnosing problems that may occur. The first variable, IG_USESOLPATH , disables the optimized code paths in the OpenGL client driver. To disable this code, set IG_USESOLPATH to 0 (IG_USESOLPATH=0). You should try this whenever you see a visual anomaly with an OpenGL program, as this helps determine the cause of the problem and may serve as a workaround to visual problems (although there may be a large performance penalty).

The second environment variable, IG_HARDWARETEX, disables the use of the hardware texturing features of the board and defaults to the software emulation. This variable should be used in any visual anomalies occurring when textures are used in an OpenGL program. To disable hardware texture support, set IG_HARDWARETEX to 0 (set IG_HARDWARETEX=0). This fix could also serve as a temporary workaround for texture problems (although there may be a great reduction in performance).