DIGITAL   (logo)

XX  GB  Hard Disk Drives

Warning/Caution

ER-XRAAM-IA. C01

Support for large IDE  Hard Disk Drives on Digital PC Desktop Systems

Due to the BIOS  limitation ….

d

On most 386, 486, and some older Pentium Desktop systems, with IDE controller, the

system may hang after installing a hard drive with a capacity greater than 528MB.

On most 486 and some older Pentium Desktop systems, with E-IDE controller,  the

system may hang after installing a HDD with capacity greater than 2 GB

Many Pentium desktop systems has limitation of  7.9 GB and 8.4GB

If your system is suspect, (check the attached Web address for list of all Digital

Desktop PCs and their IDE  Hard disk size limitation ) ,  make sure that you obtain

hard drive utility Software from the specific hard drive manufacturer, prior to the

hard disk installation. For these Internet locations, or product support phone numbers,

please contact Digital Equipment Corporation.

Before installing the drive utility software, the desktop system BIOS should set

to USER  mode 528MB with  Cylinder=1024,  Heads=16, Sector-per-track=63

Note:	The installation of an additional drive on Digital PC 3010 system  requires cable

part number FR-PCXXB-CA  for reliable data transfer for two drives configuration

Quantum http://www.quantum.com/

United States (800) 472-9799

Europe 00-44-13-44-353507

Western Digital http://www.wdc.com/

United States (714) 932-4900 or (507) 286-7900

Europe 00-31-20-44-67651

BIOS LIMITATION TO SUPPORT LARGE IDE HDD

Date: 5/98

Platform			BIOS		Limit GB

Older Platforms		N/A		N/A

Venturis 5xxx		N/A		N/A

Celebris GL 5xxx	N/A		N/A

Venturis FX		1.10		7.9

Celebris FX		1.10		7.9

Celebris GL 6xxx	1.06		7.9

Celebris GL 6xxx i2	1.01		7.9

Venturis GL 6xxx	1.06		7.9

DIGITAL PC 3100	1.10		8.4

(Venturis FX-2)

DIGITAL PC 5100	1.10		8.4

(Celebris FX-2)

DIGITAL PC 3010	1.02		8.4

DIGITAL PC 5500	1.02		7.9

(Celebris GL-2)

DIGITAL PC 5510	1.05		8.4

If the HDD size is over the native BIOS disk size limitation

then the disk manager utility (DDO=Dynamic Drive Overlay)

installation is required.

Before installing the DisK Manager, the system BIOS should

be set to "usermode"  CHS=1024x16x63

For DIGITAL PC 3010 platform, to add a second IDE drive on the

primary, a 80-conductor cable p/n: FR-PCXXB-CA is required

Information Web Sites:

http://www.windows.digital.com/ntsbuftp/TG00003A.DOC

(Bar code  here)

ER-XRAAM-IA.C01