STANDARD MICROSYSTEMS CORPORATION

Hayes LANstep Driver v2.0

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  Use EZStart to copy the Hayes LANstep driver files version 2.0

from the SuperDisk to your PC.

o  Prepare your PC's operating system so you can attach to the

network.

COPYING THE DRIVER FILES

From the Driver Installation screen, select 'Copy Files' to copy all

the files used by this driver from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory that

you specify.

PREPARING YOUR OPERATING SYSTEM

Hayes LANstep version 2.0 supports drivers for the following adapters:

o  SMC EtherCard Elite Jumpered

o  SMC EtherCard Elite Soft Configuration

o  SMC EtherCard Elite Micro Channel

Card Installation

To install one of these cards:

1. Start the LANstep INSTALL program.

2. When prompted, select the Ethernet media type and insert the

highest-numbered LANstep Network Driver Disk you have.

3. Choose the appropriate SMC adapter from the menu.

4. When INSTALL prompts you, enter the correct values for the

network card settings, if necessary.

LANstep automatically updates your PROTOCOL.INI, CONFIG.SYS,

and LANSTEP.INI files

To install an SMC card other than one listed above (such as one for

which LANstep does not include a driver), follow the steps above, but

choose "Other network cards" when INSTALL prompts you with a list of

network cards. Read the information screen, then press Enter to return

to the DOS prompt.

Workstation Configuration

For a workstation, follow these steps:

1. Use any text editor to include the following lines in your

CONFIG.SYS file:

DEVICE=C:\LANSTEP\PROTMAN.SYS /I:C:\LANSTEP /E:C:\LANSTEP\PROTMAN.EXE

DEVICE=C:\LANSTEP\SMC8000.DOS

2. Modify the PROTOCOL.INI file in the \LANSTEP subdirectory

on your PC's hard disk so it resembles the following:

[PROTOCOL MANAGER]

DRIVERNAME = PROTMAN$

[LANSTEP]

DRIVERNAME = LANSTEP$

INTERRUPT = 3

BINDINGS = SMC8000

[SMC8000]

DRIVERNAME = SMC8000$

IOBASE = 0x280

LANstep requires an INTERRUPT = entry in the [LANSTEP] section of

the PROTOCOL.INI file that matches the Ethernet adapter's

IRQ setting. If you change the interrupt on the adapter, remember

to change the [LANSTEP] section of the PROTOCOL.INI file. (This is

a requirement on workstations only, not on servers.)

Server Configuration

For a server, follow these steps:

1. Use any text editor to include the following lines in the

LANSTEP.INI file in the \LANSTEP subdirectory on your PC's hard

disk:

[SERVER]

PROTMAN = C:\LANSTEP\PROTMAN.OS2

NDISDRVR = C:\LANSTEP\SMC8000.OS2

2. Modify the PROTOCOL.INI file in the \LANSTEP subdirectory

on your PC's hard disk so it resembles the following:

[SMC8000]

DRIVERNAME = SMC8000$

IOBASE = 0x280

RAMADDRESS = 0xD000

IRQ = 3

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, Elite Series, EliteView, EZStart, SuperDisk, and UltraChip

are trademarks of Standard Microsystems Corporation.  Other products

and company names are registered trademarks or trademarks of their

respective holders.