SMCPWRN NDIS 3.1 miniport driver for Intel Windows NT [tm] 3.51

Version v4.15

31-Dec-1996

1. Copyright Notice:

First Edition, April 1994

Updated, December 1994

Updated, February 1996

Updated, April 1996

Digital Equipment Corporation makes no representations that

the use of its products in the manner described in this

publication will not infringe on existing or future patent

rights, nor do the descriptions contained in this

publication imply the granting of licenses to make, use, or

sell equipment or software in accordance with the

description.

Copyright Digital Equipment Corporation 1994.

All rights reserved.

Do not redistribute the driver files.

Windows, Windows NT, and Windows95 are trademarks of

Microsoft Corporation.

Microsoft, MS, MS-DOS are registered trademarks of

Microsoft Corporation.

Digital Semiconductor's, Digital, Digital Semiconductor are

trademarks of Digital Equipment Corporation.

All other trademarks and registered trademarks are the

property of their respective holders.

2. Overview:

3. Requirements:

A.  Hardware Requirements:

EtherPower PCI adapter

B.  Software Revision Requirements:

Windows NT Version 3.51 or later

4. Files and directories:

---------------------------------------------------------------------

INSTALL.DOC         This file.

---------------------------------------------------

OEMSETUP.INF        The driver's setup file for Windows NT

---------------------------------------------------

SMCPWRN.SYS         The driver's executable

---------------------------------------------------

SMCPWRN.DBG         The driver's symbol table

---------------------------------------------------------------------

5. Installing the Hardware Boards:

A. Installing SMC's EtherPower PCI Ethernet Controller:

Refer to SMC's EtherPower the board's User Guide.

6. Installing the SMCPWRN Driver on Windows NT:

1. From the Windows NT Main Menu, select the Control Panel icon.

2. From the Control Panel menu select the Network icon.

If a dialog box asks you "Do you want to install NT

Network?" select YES and Continue. Then place

the Windows NT CDROM in the CD-ROM reader.

Select the drive that matches the configuration of your system

and complete the install.

3. You will see several messages. To add a network adapter, select Yes

and then Continue

4. For each additional message, select continue.

5. From the Add Network Adapter dialog box, scroll through the menu to

select Other.

6. When the Insert Diskettes menu prompts you for a disk, insert the

SMCPWRN driver installation diskette for Windows NT into drive A.

Then select OK.

7. Select your option

[SMC 8432 EtherPower PCI Ethernet Adapter without Boot ROM socket (8432)]

[SMC 8432 EtherPower PCI Ethernet Adapter with Boot ROM socket (8432E)  ]

[SMC 8434 EtherPower PCI2 Ethernet Adapter (8434)                       ]

[SMC 9332 EtherPower10/100 PCI Fast Ethernet Adapter (9332DST)          ]

[SMC 9332 EtherPower10/100 PCI Fast Ethernet Adapter (9332BDT)          ]

[SMC 9332 EtherPower10/100T4 PCI Fast Ethernet Adapter (9332BVT)        ]

[SMC 9334 EtherPower10/100 PCI Fast Ethernet Adapter (9334BDT)          ]

[SMCDC21143 EtherPower10/100 PCI Fast Ethernet Adapter                  ]

from the Select OEM Option box, then select OK.

8. Modify the default distribution path in the Windows NT

Setup box to point to the correct drive and path, then click

on [Continue]

9. For PCI options, a Dialog box pops up to select the:

DataRate:

------------------------------------------------------------------------

SMC 8432

-------------------------------------------------------------------------

o AutoDetect                    In AutoDetect mode the driver

checks first the Twisted Pair port.

If the Link Test fails the driver

checks the BNC port. If no carrier is

detected the driver switches to the AUI

port.

o Twisted_Pair (10BaseT)

o Twisted_Pair Full_Duplex      To connect to Full Duplex hubs

o Twisted_Pair No_Link_Test     To connect hubs not supporting

the Link Integrity Test

o BNC (10Base2)                 BNC connector

o AUI (10Base5)                 15_pin connector

------------------------------------------------------------------------

SMC 9332

-------------------------------------------------------------------------

o AutoDetect                    In AutoDetect mode the driver

checks the 100BaseTx and 10BaseT ports.

If no link is detected the driver

selects the 100BaseTx port.

o 100Mbps                       To connect 100BaseTx Ethernet hubs

o 100Mbps Full_Duplex           To connect 100BaseTx Full Duplex hubs

o 10Mbps                        To connect 10BaseT Ethernet hubs

10. Upon completion, the selected adapter is added to the

Installed Adapter Cards list of the Network Setting box

(the number prefixing the adapter is the adapter number).

11. Click on [Close] to complete the Network Setting

initialization.

12. If needed, modify the values in the registry (see section 7 below).

13. Shutdown and reboot the system.

7. Parameters

To overwrite the driver default values, add or edit the

following values in the Registry SMCPWRN<adapter_number> Parameters

subtree:

CacheLineSize = REG_DWORD { 16 | 32 | 64 | 128 }

16 bytes

32 bytes (default)

64 bytes

128 bytes

InterruptMitigation = REG_DWORD  {0 | 1} (DC21140,DC21142)

0  Disabled (default)

1  Enabled

Enable/disable the Interrupt Mitigation

algorithm.

InterruptThreshold = REG_DWORD {value} (default: 500)

Maximum number of interrupt/second allowed before

the Receive and Transmit interrupts get masked.

Meaningfull only when InterruptMitigation is enabled.

FrameThreshold = REG_DWORD {value} (default: 400)

Minimum number of processed frame/second allowed

before the Receive and Transmit interrupts get

unmasked.

Meaningfull only when InterruptMitigation is enabled.

MapRegisters = REG_DWORD {4,8,16,32,64}  (default: 64)

Number of map registers allocated

by the adapter.

NetworkAddress = REG_SZ { xx-xx-xx-xx-xx-xx }

Mandatory if no Serial ID PROM is plugged in

Overwrite the Serial ID PROM if plugged in

NetworkAddress 00-00-00-00-00-00 is ignored

NwayProtocol = REG_DWORD { 0 | 1 }  (DC21041 and

DC21142 Rev1.0,1.1 only)

0  Disabled

1  Enabled  (default)

Enable/disable the 10BT/10BT_FD Nway

medium negotiation in Autosense mode.

ReceiveBuffers = REG_DWORD {8..64} (default: 16)

Number of buffers allocated by the

driver in the Receive ring.

SoftwareCRC = REG_DWORD { 0 |  1 }  (DC21040 rev 1.x, 2.0 & 2.2 only)

0 The CRC is generated by the adapter

1 (default) The CRC is generated by software for

transmit packets bigger than the transmit threshold.

NOTE: This option affects performances and could be

disabled if the adapter does not report transmit

underruns in its statistic counters

UnderrunRetry = REG_DWORD  number_of_retries  (default=2)

Maximun number of retransmission attempts of

an underrun packet.

UnderrunThreshold = REG_DWORD  number_of_underruns (default=10)  (DC21140 and DC21142 only)

Maximun number of underrun occurences before the

controller switches to the next threshold value.

The following values allow to overwrite the SMCPWR default setting.

Refer to the Digital Semiconductor's DC21040, DC21041, DC21140\DC21140A, DC21142,

DC21143 Engineering specifications for detailled description:

BackPressure = REG_DWORD { 0 | 1 }

CSR6<BP>    0  Disabled (default)

1  Enabled

BurstLength = REG_DWORD  { 0 | 1 | 2 | 4 | 8 | 16 | 32 }

CSR0<PBL>   0  unlimited     (default) - (not 21040, 21041 & 21140 rev1)

1  doubleword

2  doublewords

4  doublewords

8  doublewords

16  doublewords   (default for 21040, 21041 & 21140 rev1)

32  doublewords   (not 21040 & 21041)

CaptureEffect = REG_DWORD { 0 | 1 }

CSR6<CA>    0  Disabled (Default)

1  Enabled

FifoArbitration = REG_DWORD { 0 | 1 }

CSR0<BAR>   0 Rcv/Txm Arbitration (default)

1 Round_Robin

SnoozeMode = REG_DWORD { 0 | 1 }  (DC21041 only)

CFDA<SM>    0 Disabled (default)

1 Enabled

StopBackoffCounter = REG_DWORD { 0 | 1 }

CSR6<SB>    0  Disabled (Default)

1  Enabled

StoreAndForward = REG_DWORD { 0 | 1 }  (DC21140 only)

Select the StoreAndForward mode if high

PCI latencies causes 100Mbps Transmit Underruns

CSR6<SF>    1 Enabled

0 Disabled (default)

TransmitThreshold = REG_DWORD { 72 | 96  | 128 | 160 }

Transmit Threshold value for the 10Mbps media.

CSR6<TTM,TR>  72 bytes

96 bytes (default)

128 bytes

160 bytes

TransmitThreshold100 = REG_DWORD { 128 | 256 | 512 | 1024 }

Transmit Threshold value for the 100Mbps media.

Meaningless when StoreAndForward is set

CSR6<TTM,TR> 128 bytes

256 bytes

512 bytes (default)

1024 bytes

8. Driver Revision number

To retrieve the driver revision number either

select the file DC21X4.SYS and open the File Properties applet

or

query the Registry:

HKEY_LOCAL_MACHINE\

Software\DigitalEquipmentCorporation\DC21X4\CurrentVersion

MajorVersion

MinorVersion

9. Error messages

The error messages are logged in the Entry Log and can be seen using the Entry

Log Viewer (in WNT's Administration Tools group).

Each error message has a standard Ndis error code and an driver's additional

code.The additional codes are:

DC21X4_ERRMSG_REGISTRY              0x01

DC21X4_ERRMSG_ALLOC_MEMORY          0x02

DC21X4_ERRMSG_SROM                  0x03

DC21X4_ERRMSG_MEDIA                 0x04

DC21X4_ERRMSG_LOAD_CAM              0x05

DC21X4_ERRMSG_SYSTEM_ERROR          0x06

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT    0x07

9.1 Driver initialization errors:

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_REGISTRY

The AdapterType Registry's key is missing or its value is

unsupported by the adapter board

o NDIS_ERROR_CODE_ADAPTER_NOT_FOUND

No board matching the AdapterCFID Registry's key value was found

plugged into the PCI bus or in the slot specified in the Registry

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_ALLOC_MEMORY

Not enough memory to allocate space for the Adapter data block

o NDIS_ERROR_CODE_INVALID_VALUE_FROM_ADAPTER

DC21X4_ERRMSG_SROM

Invalid data read from the adapter's serial ROM

o NDIS_ERROR_CODE_NETWORK_ADDRESS

No readable burnt_in or software configured station address

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_MEDIA

The selected media port is not supported by the board.

o NDIS_ERROR_CODE_INTERRUPT_CONNECT

Interrupt initialization failed

o NDIS_ERROR_CODE_TIMEOUT

DC21X4_ERRMSG_LOAD_CAM

DC21X4 CAM initialization failed

9.2 Driver run time errors:

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_SYSTEM_ERROR

System_Error interrupt (bus parity error)

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT

Transmit Jabber timer expired