NetWare Support Pack

µ §

Document #515-0002385							August 1994

## Table of Contents

µTable of Contents	2

Overview	3

Certified Hardware	3

LAN Adapters	3

AT&T NetWare Drivers	4

Drivers by Class Number	4

Loading Drivers	4

Command Line Parameters supported by Host Adapter Drivers	4

Disk Drives Tested with the NCR SCSI Drivers	6

Tape Drives Tested	6

CD-ROM Drives Tested	6

ASPI Tape Backup/Restore Utilities Tested	7

Installing SBACKUP Tape Support for NetWare Version 3.11	7

Running SBACKUP on NETWARE Version 3.12	7

Installing NetWare from CD-ROM	8

Memory Requirements	9

Support Contacts	9

Clients and Servers	9

NetWire Forum on CompuServe	9

FaxLine	9

Bulletin Board System	9

Restrictions and Troubleshooting	11

Optimization	13

Novell/AT&T Global Information Solutions Relationship	14

SES	14

Technical Support Alliance (TSA)	14

Certified NetWare Engineers (CNE)	15

Novell Professional Developer Program	15

Self Certifying Lab	16

## Overview

The AT&T Global Information Solutions NetWare Support Pack is provided as a reference for support of NetWare products on AT&T hardware platforms.  It offers information on certification, installation and configuration, and solutions to common problems.

## Certified Hardware

A complete list of Novell certified hardware systems, subsystems, adapters and peripherals released under the NCR logo and recent releases under the AT&T logo is available on WPD Clemson's FaxLine system (refer to FaxLine later in this document) .  For additional information on a specific entry, refer to the Novell bulletin number listed.  Bulletins are shipped on Novell's Netware Support Encyclopedia.  Recent bulletins appear on Clemson's FaxLine system.

## LAN Adapters

System certification requires the following LAN adapters to be tested with a system:

| ISA | Novell NE1000 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Novell NE2000 |  |  |  |
|  | SMC ARCNet PC130 |  |  |  |
|  | IBM TR 16/4 |  |  |  |
|  | AT&T Global Information Solutions WaveLAN |  |  |  |
|  | AT&T Global Information Solutions LANPacer+ |  |  |  |
|  | Intel EtherExpress |  |  |  |
|  | AT&T Global Information Solutions TR 16/4 |  |  |  |
| MCA | Novell NE/2 |  |  |  |
|  | Novell NE2-32 |  |  |  |
|  | IBM TR 16/4 /A |  |  |  |
|  | IBM TR II/A |  |  |  |
|  | IBM TR 16/4 Bus Master |  |  |  |
|  | 3Com 3C523 |  |  |  |
|  | SMC ARCNet PS110 |  |  |  |
|  | AT&T Global Information Solutions TR 16/4 |  |  |  |
|  | AT&T Global Information Solutions WaveLAN |  |  |  |
|  | Thomas Conrad TC3046-ST (w/SFTIII as MSL) |  |  |  |  |
|  | PlainTree PDP403A (w/SFTIII as MSL) |  |  |  |  |

Other LAN adapters are tested during in-house integration testing and appear in the product release documentation.

## AT&T NetWare Drivers

WPD Clemson provides drivers for SCSI host adapters on its Micro Channel platforms.  These drivers are provided on a support diskette shipped with a platform and on Clemson's bulletin board.

WPD Clemson is improving the functionality of the SCSI drivers it is releasing.  These improved drivers will support the Advanced SCSI Peripheral Interface (ASPI).  ASPI is required to support popular third party applications (e.g. Cheyenne ArcServe, LeGato NetWorker for NetWare).  Both tape and CD-ROM are supported through the ASPI and non-ASPI versions of the drivers.

CD-ROM support, allowing a CD-ROM to appear as a virtual drive, is a new addition to these drivers.   NetWare v3.12 or v4.01 installation does not require this support.

## Drivers by Class Number

The SCSI host adapter present in Micro Channel AT&T systems can be determined by the system's diagnostic diskette or the class number on the serial number label.  Use the following matrix to identify the SCSI driver by platform.

| Class Number | Adapter | NetWare 3.1x | NetWare 4.x |
| --- | --- | --- | --- |
| 3420, 3421, 3320, 3321, 3220, 3221, 3314 | 53C90 | NCR9X_3.DSK | NCR9X_4.DSK |
| 3350, 3355, 3410 | 53C94 | NCR9X_3.DSK | NCR9X_4.DSK |
| 3432, 3433, 3434, 3437, 3355, 3345, 3445, 3447 | 53C700 | NCR700_3.DSK | NCR700_4.DSK |
| 3360, 3430 | 53C710 | NCR710_3.DSK | NCR710_4.DSK |

## Loading Drivers

During the loading of any of the above host adapter drivers, the NCR Advanced SCSI Programming Interface (ASPI) Manager Driver will be auto loaded.  This driver provides the ASPI support required by applications implementing ASPI.  For NetWare v3.1x, NCRASPI3.DSK will be loaded; for NetWare v4.x, NCRASPI4.DSK will be loaded.

Load the appropriate driver once for each SCSI controller.  NOTE:  If the NCR 53C710 Daughter Card (Kit or Feature Number 3360-K/F391 and 3360-K/F392), is being used, there is no need to load the NCR 53C710 driver an additional time for this card.  The driver for the baseboard or host adapter NCR 53C710 will also provide support for the daughter card.

## Command Line Parameters supported by Host Adapter Drivers

The Host Adapter Drivers support the following command line parameters used with the load command (ex: LOAD NCR9X_4.DSK slot=5 /ns):

/i		Displays driver information.  Setting this flag will display information regarding the

driver's current modes of operation.

/ns		Disables synchronous data transfers with drives that support synchronous data transfers,

default is synchronous data transfers enabled for the NCR710_x.DSK.

/nd		Disables drive disconnect from the SCSI bus, drive disconnect is enabled by default.

Disconnect enables the driver to send I/O requests to multiple SCSI devices at the same

time.  Use only with multiple SCSI devices, especially with drive mirroring.  NOTE:  If

you want command queuing, you MUST have disconnect enabled.

/nq		Disables command queuing with drives that support command queuing, default is

command queuing enabled.  Command queuing enabled will result in the driver sending

multiple I/O requests to SCSI drives that support command queuing.  NOTE:  If /nd is

specified, then command queuing is also disabled because command queuing requires

the use of disconnect.

/nsg		Disables Scatter/Gather implementation, default is Scatter/Gather enabled.

Scatter/Gather causes the driver to queue up and combine I/O requests and send them

out at one time to the drive.  This is beneficial when large file transfers are being

implemented.

/m:(5 - 250)	Driver Memory Allocation, default is set to 100.  Setting this parameter will adjust the

amount of memory used by the driver for SCSI Request Blocks.  The range is 5 to 250

SCSI Request Blocks.  (ex: LOAD NCR9X_3.DSK /m:250).

## Disk Drives Tested with the NCR SCSI Drivers

The following is a list of the SCSI disk drives that have been tested with the NetWare v3.1x and v4.x device driver.  The driver should work with other disk drives, but it has only been tested with those listed

below.

| Drive Vendor | Drive Model | Drive Capacity | Form Factor |
| --- | --- | --- | --- |
| Conner | CP3100 | 100MB | 3.5-inch (HH) |
| Conner | CP30540 | 545MB | 3.5-inch (HH) |
| Quantum | LPS-105S | 105MB | 3.5-inch (HH) |
| Quantum | LPS-120S | 120MB | 3.5-inch (HH) |
| Maxtor | LXT-200S | 200MB | 3.5-inch (HH) |
| Maxtor | LXT-213S | 200MB | 3.5-inch (HH) |
| Maxtor | LXT-340S | 340MB | 3.5-inch (HH) |
| Maxtor | LXT-535S | 535MB | 3.5-inch (HH) |
| Maxtor | 7213-SCSI | 200MB | 3.5-inch (HH) |
| DEC | DSP-3105S | 1000MB | 3.5-inch (HH) |
| CDC-Imprimis | WREN IV-327 | 327MB | 5.25-inch (FH) |
| Imprimis | 94601-15 | 992MB | 5.25-inch (FH) |
| Maxtor | XT-8760S | 760MB | 5.25-inch (FH) |

## Tape Drives Tested

The following is a list of the SCSI tape drives that have been tested with the NetWare v3.1x and v4.0x device drivers.

| Drive Vendor | Drive Model | Media From Factor | Drive Form Factor |
| --- | --- | --- | --- |
| HP | 35470A | 4mm DAT | 3.5-inch (HH) |
| Wangtek | 5525ES | 0.25-inch Cartridge | 5.25-inch (HH) |

## CD-ROM Drives Tested

The following is a list of the SCSI CD-ROM drives that have been tested with the NetWare v3.1x and v4.0x device drivers.

| Drive Vendor | Drive Model |
| --- | --- |
| Sony | CDU-541 |
| Sony | CDU-561 |
| Toshiba | XM-3301TA |

## ASPI Tape Backup/Restore Utilities Tested

The following is a list of Tape Backup/Restore Utilities requiring ASPI support that have been tested with the NetWare v3.1x and v4.0x device drivers.

Cheyenne ARCserve for NetWare Windows Edition Version 5.0

Cheyenne ARCserve for NetWare Version 4.0

Legato Networker NetWare Version, Release 2.10 (only supports NetWare v3.1x)

NOTE:  During the installation of ASPI utilities, if prompted to choose a host adapter, driver, or ASPI manager selection, choose the Adaptec ASPI SCSI Manager or a generic ASPI SCSI Manager.

## Installing SBACKUP Tape Support for NetWare Version 3.11

This section explains how to install support for the HP 35470A 2GB DAT Tape Drive and the Wangtek 5525ES 525MB Streaming Tape Drive for NetWare's SBACKUP utility.

To perform Server Tape Backup and Restore, the latest Novell supplied SBACKUP utility needs to be used.  This utility may be acquired from  Novell's Netwire service on Compuserve, or the AT&T Global Support Center Bulletin Board.

To download SBACKUP and the associated files from the Bulletin  Board, use the following information:

| Telephone Number | 1-803-843-1811 |
| --- | --- |
| Speed | 28.8 KBaud or less |
| Data bits | 8 |
| Parity | None |
| Stop bits | 1 |

Download NCRBACK.EXE and execute the self-extracting file.  From a workstation connected to the Netware 3.11 server that will perform the Server Tape Backup and Restore, copy the following files extracted from NCRBACK.EXE to the file server's SYS:SYSTEM subdirectory.

SBACKUP.NLM

DIBIDAI.NLM

TSA.NLM

TSA-311.NLM

SIDR.NLM

Also copy DIBI2$DV.DAT from NCRBACK.EXE to SYS:SYSTEM\DIBI subdirectory.

## Running SBACKUP on NETWARE Version 3.12

During the execution of SBACKUP, the following menu will appear:

Select Device Driver

==================

Wangtek Tape Driver

DIBI-2 DAI Tape Driver

DC00 DIBI-2 Tape Driver

To execute the AT&T tape driver for both the Wangtek 5525ES and HP 35470A tape drives, you must select the second option, DIBI-2 DAI Tape Driver.

## Installing NetWare from CD-ROM

This section describes how to create a DOS boot diskette or DOS partition on a hard drive to allow NetWare installation from a CD-ROM.

NOTE:  The Global Support Center Bulletin Board may be used to download the latest releases of the Novell Support Diskette and the NCR Software Support Diskette.  The latest Novell Drivers will be on the Novell support diskette.  The NCR Software Support Diskette will include all SCSI drivers for MS-DOS as well as all video drivers.

1.	Boot the system with a DOS boot diskette, i.e. DOS installation diskette and go to the command

prompt "A>".

2a.	If creating a DOS boot diskette, format a 3.5-inch high-density diskette as a system diskette to

create a DOS.

FORMAT B:/S

Once prompted to insert a diskette into drive B, remove the DOS diskette from the drive and

insert a blank diskette.  Press ENTER.

2b.	If creating a DOS partition on the hard drive, create a DOS partition (5 MB minimum) on the

hard drive and format the partition.

3.	Insert the Software Support Diskette into the flex drive and, copy the DOS drivers from the

Software Support Diskette to the newly formatted blank diskette (for boot diskette) or the C: drive

(for hard drive installation).

COPY A:\SCSI\DOS\DRVDRVRS.EXE B: (boot diskette)

-or-

COPY A:\SCSI\DOS\DRVDRVRS.EXE C: (hard drive installation)

4.	DRVDRVRS.EXE is a self extracting file that, when executed, will extract the DOS SCSI

drivers.  From the DOS prompt type:

DRVDRVRS.EXE

5.	Add the following lines to CONFIG.SYS.  Delete any memory managers that may be installed.

DEVICE=CAMC9X.SYS *

DEVICE=DISKIDD.SYS

DEVICE=CD-TOSH.SYS /D:MSC001

-or if a Sony CD-ROM is installed-

DEVICE=CD-SONY.SYS /D:MSC001

*  This line will depend on the SCSI host adapter present in the NCR system.  Refer to the "How

to Prepare for Installation" section of this document to determine what SCSI adapter is used.

CAMC9X.SYS is for 53C90 and 53C94, CAMC700.SYS is for 53C700 and CAMC710.SYS

is for 53C710.

6.	Add the following line to AUTOEXEC.BAT.

MSCDEX.EXE /D:MSC001 <Enter>

7.	Reboot the computer by pressing the Ctrl, Alt, and Del keys at the same time.

8.	Follow the NetWare CD Installation Procedures.

## Memory Requirements

NetWare memory requirements are determined by disk capacity and installed software.  The system administrator should determine the needs for the server based upon the following steps:

1)	NetWare v3.12 requires a base of 4 MB, v4.01 requires a base of 6 MB.

2)	An additional 2 MB is required if any of the following are installed:

Additional NetWare Products

Print server (PSERVER.NLM)

Any of the following NetWare Loadable Modules

CLIB.NLM, STREAMS.NLM, BTRIEVE.NLM

3)	Multiply the amount of system disk space (in MB) by .008, add to the memory requirements

above.

4)	Add 1 to 4 MB for additional cache buffer RAM to optimize performance (the more RAM added,

the better performance).

## Support Contacts

### Clients and Servers

Pre-Sales:  Pre-Sales Assistance Center, 1-800-RING PAC

Post-Sales:  Atlanta System Support Center 1-800-531-2222

### NetWire Forum on CompuServe

Frequently asked questions can be found on NetWire.  This forum is monitored by the members of TSA.  Anyone with access to CompuServe may access NetWire.

### FaxLine

WPD Clemson maintains a fax-back system.  This system has marketing and general information.  Clemson's FaxLine system is accessed by dialing 1-800-438-4282 or VP 624 -4282.

### Bulletin Board System

Clemson maintains a bulletin board with NetWare drivers and associated NetWare files.   To access the BBS use the following information:

| Telephone Number | 1-803-843-1811 |
| --- | --- |
| Speed | 28.8 KBaud or less |
| Data bits | 8 |
| Parity | None |
| Stop bits | 1 |

## Restrictions and Troubleshooting

*NOTE*:

Multiple ASPI Drivers may not be supported

.

-----------------------------------------------------------------------------

Symptom:   NCRASPIx.DSK WILL NOT LOAD

Resolution:     NCRASPI3.DSK and NCRASPI4.DSK will not load if another ASPI Manager Driver is already loaded, such as Adaptec's ASPITRAN.DSK.  Unload the other ASPI Manager Driver and then load the appropriate NCRASPIx.DSK driver.

-----------------------------------------------------------------------------

Symptom:     CANNOT LOAD ANOTHER VENDOR'S HOST ADAPTER DRIVER

Resolution:    Contact the Host Adapter Vendor for support or a new driver.

-----------------------------------------------------------------------------

Symptom:    HOT-PLUGGED DRIVE IS NOT RECOGNIZED BY NETWARE OR REMOVED DEFECTIVE DRIVE  IS NOT DELETED FROM THE LIST OF DEVICES

Resolution:  Replacing drives while the system is powered on is called "hot plugging".

Hot-plugging drives is NOT recommended.

-----------------------------------------------------------------------------

*NOTE*:

Installation requires a 20MB DOS partition.

-----------------------------------------------------------------------------

Symptom:  During installation INSUFFIECIENT SPACE IS REPORTED BEFORE ALL FILES ARE INSTALLED

Resolution:  Novell's installation procedure does not allocate sufficient space for the DOS partition.  Using FDISK create a 20MB DOS and install DOS before continuing with Novell's instructions.

-----------------------------------------------------------------------------

Symptom:   DIFFICULTY CREATING NETWARE VOLUMES

Resolution:     If there is difficulty creating NetWare volumes during the NetWare install process, check the NCR 53C710 SCSI Host Adapter for an older  version (Assembly Number 515-0000841A) installed in the system.  If you the NCR 53C710 SCSI Host Adapter is this version or earlier, call the AT&T Support Center at (800) 531-2222 to upgrade the  NCR 53C710 SCSI Host Adapter.

-----------------------------------------------------------------------------

Symptom:   ARCSERVE DOES NOT RECOGNIZE ANY DEVICES ON THE ADDITIONAL HOST ADAPTER

Resolution:   ARCserve currently only supports devices on a single host adapter.  If the tape drive is connected to any secondary host adapters, reconnect it to the host adapter which is loaded first.  The host   adapter with the tape drive may be loaded prior to any other host adapter by specifying the slot number of that host adapter with the LOAD command line parameter,  "slot=x" (where x is the slot of the host adapter with the tape drive).

-----------------------------------------------------------------------------

*NOTE:*

Symptom:   ARCSERVE CAUSES THE SYSTEM TO LOCK-UP WHEN USING A WANGTEK TAPE DRIVE

Resolution:   ARCserve only supports Wangtek tape drives that implement the SCSI-II command set.  Use the NCR Diagnostics Diskette to determine the revision of the Wangtek tape drive installed in the system.  If the Wangtek 5525 quarter inch tape drive's firmware revision is 16H12, call the AT&T Global  Support Center at (800) 531-2222 for further information.

-----------------------------------------------------------------------------

* ARCSERVE Requires two logical connections and is equivalent to  two users logged on to the server.

-----------------------------------------------------------------------------

Symptom:   INITIAL MOUNT OF CDROM CAUSES WRITE ERROR MESSAGES ON THE SYSTEM CONSOLE  SCREEN

Resolution:  When CDROM media is mounted under NetWare v4.01, error messages are presented to the user on the System Console screen.  These messages can be ignored and do not affect system integrity or performance.

-----------------------------------------------------------------------------

Symptom:   CACHE MEMORY ALLOCATOR OUT OF AVAILABLE MEMORY (message at the console screen during backup.)

Resolution:   Use the SET command to increase the amount of reserved buffers allocated below 16 megabytes for SBACKUP use.  Add the following command line in  STARTUP.NCF file:

SET RESERVED BUFFERS BELOW 16 MEG = 100 (or higher)

-----------------------------------------------------------------------------

Symptom:  MESSAGE PASSING TEST FAILED TO RUN WITH FEWER THAN 16 RECEIVE BUFFERS

Resolution:  'Minimum Packet Receive Buffers' is a Netware communications parameter which determines the minimum number of packet receive buffers that the OS can allocate.  The default number is 10; it can be set to anything from 10 to 1,000.  For some NICs, if it is set too low, clients can be starved and sometimes drop off.

Enter 'set' at the system console to look at and change SET parameters.  'Minimum Packet Receive Buffers' can also be set in STARTUP.NCF ("set minimum packet receive buffers = X"); other parameters can be set in either STARTUP.NCF or AUTOEXEC.NCF.

Cogent E/MASTER III ethernet adapters require minimum packets to be set to 6 for each NIC to load the driver.  However, the test fails to run unless a total of 16 is specified.

-----------------------------------------------------------------------------

*NOTE:*

DURING NETWORK INSTALLATION METHOD OF NETWARE V4.01, COPY THE DRIVERS TO

THE DOS PARTITION WITH THE NETWARE FILES.

-----------------------------------------------------------------------------

*NOTE:*

16MB memory limit on 3445/3447 architecture

-----------------------------------------------------------------------------

Symptom:  NCR Token Ring Adapter fails.  This is only one of many symptoms - see below.

Resolution:  Put adapter in DIO Slave mode from DMA Bus Master.

For Novell there is 16MB memory limitation on these 3445/47 platforms due to an incompatibility with some 32bit bus master cards which access DMA above 16MB.     Failing cards include:

IBM Bus Master Server Adapter

Madge Smart Ring Node MC/32

DCA Irmatrack Tokenring

There are also some 16-bit boards (such as Adaptec) whose drivers have a load-line switch to inform it that there is more than 16MB in the machine.  The "autoregister" command is needed.  However, then the disk driver must be loaded below the 16MB and the memory above 16MB is registered manually.

Adaptec's 16-bit  bus mastering boards, 154X, 164X, and 174X (standard mode only) will perform bus mastering with more than 16MB of registered memory if you use the "above16=y" switch.  The driver must load below the 16MB address in memory.  The only way to guarantee that condition (at least on EISA machines) is to bring up the server with the following order:

1.   In STARTUP.NCF:

"SET AUTO REGISTER MEMORY ABOVE 16 MEGABYTES=OFF

"SET RESERVED BUFFERS BELOW 16 MEG=40 (Add 20 buffers for each

additional SCSI device supported by the 1540 driver with a

minimum of 100 buffers for SBACKU.NLM and ARCSERVE.NLM.)

2.   In AUTOEXEC.NCF:

file server name <servername>

ipx internal net <address>

load AHAXXX above16=y prot=<port address> int=<interrupt>

register memory 1000000 <memory above 16MB in hex>

mount<volumes>

load <applicable NLMs>

set <applicable parameters>

* Note:  The AUTOEXEC.NCF  must be in the same boot

directory as the SERVER.EXE.  This means you should copy the

AUTOEXEC.CFG file to the boot drive or diskette and then delete or

rename it in the SYS:SYSTEM volume on the server.

*NOTE:*

The 6298 RAID subsystem is a fast and wide SCSI requiring a differential connector.  The only WPD product which supports fast SCSI is the 3430.  It does not support wide SCSI.  A daughter card may  be added to the 53C710 adapter for differential.

-----------------------------------------------------------------------------

*NOTE:*

Popular external UPS's are supported by a NetWare NLM in the Red Box.  Internal UPS are not supported by this NLM and will require development work.

-----------------------------------------------------------------------------

## Optimization

It is difficult to find two networks which are exactly alike, and therefore, it is difficult to present absolute methods for optimizing all networks.  This section is a list of suggested areas to investigate to improve network performance.   Several references are available on this subject as well as a three part articles in NetWare Connection, the NUI's magazine. (Nov/Dec 1992, Jan/Feb 1993, Mar/Apr 1993 issues, author, Stephen Campbell).

For clients investigate:

processor speed of the client

transfer speed of network interface card relative to traffic on the network

evaluate efficiency of network adapter driver

Packet Burst mode may improve performance; weigh against resources required to run PBURST

Use DOS 5.0 or later to save conventional RAM

Use NetWare Expanded or Extended Memory Shells

Note:  Universal Client (NetWare Client for DOS/Windows includes intelligent memory 		management)

Use Extended or Expanded Memory to run device drivers and TSRs

Minimize number of drive mappings; create dynamically and delete when no longer need

Use efficient menu systems when needed

Minimize use of unnecessary TSR's

NET.CFG MAX FRAME SIZE for token ring not set larger than 4KB

IPX PACKET SIZE LIMIT for Ethernet set to 1500 bytes

Windows swap files on local hard drive or RAM

NET.CFG FILE HANDLES = 60 for Windows

For NetWare 3.x investigate:

Disk duplexing to speed up disk I/O

Disk spanning  to speed up disk I/O

SCSI disconnect feature support for disks and host adapters

Determine largest allowable packet size for network

Packet Burst mode may improve performance for WAN, large internetwork, large network files

Add server memory

Increase memory for file caching

Match cache buffer size and volume block size

Use Network Interface Card with best performance

Use bus mastering adapters

Check NetWire for performance utilities to download.

ALWAYS TEST CHANGES THOROUGHLY BEFORE IMPLEMENTING THEM

## Novell/AT&T Global Information Solutions Relationship

AT&T Global Information Solutions  and Novell have worked together on several initiatives.  Listed below are areas where Novell and AT&T Global Information Solutions have cooperated in the past and continue to explore possible future joint opportunities.

### SES

Several AT&T Global Information Solutions plants participate in a Strategic  Engineering Agreement and deliver products for use on NetWare.  In this program Novell assigns a dedicated Strategic Engineer  to AT&T Global Information Solutions products.  Clemson coordinates the contract for this agreement.

### Technical Support Alliance (TSA)

WPD Clemson  participates in the Technical  Support Alliance (TSA).  Under this program a problem  call  to  Novell  will  link customers to participating vendors directly.  TSA members have agreed to exchange information about their products with each other allowing support technicians to  be more aware of other companies technologies.

The following is a list of current TSA members:

3COM

Adaptec

ALR

Apple

AST Research

AT&T Global Information Solutions

Borland

Cisco Systems

Compaq

DaVinci Systems

Dell Computer

Digital Equipment

Gupta

Hewlett-Packard

IBM

Intel

Lotus

Microsoft

NEC Technologies

NetFRAME Systems

Novell

Olivetti

Oracle

Shiva

Standard Microsystems

Storage Dynamics

Sun  Microsystems

SynOptics Communication

Ungermann-Bass

UNISYS

Walldata

WordPerfect

Zenith Data Systems

### Certified NetWare Engineers (CNE)

AT&T Global Information Solutions  has certified NetWare instructors (CNI's) and has trained about 200 AT&T Global Information Solutions employees.  About 70 have achieved certification (CNE status) through the AT&T Global Information Solutions program.  Other AT&T Global Information Solutions employees have gained training and certification through other Novell certified training centers.  Europe has about 45 Novell trained personnel.

### Novell Professional Developer Program

AT&T Global Information Solutions developers  belong to  Novell's  Professional  Developer Program  which provides products such as compilers  and  software tool  kits  as  well  as services  to  those  developing  NetWare compatible products. Developers attend Novell classes as well  as Novell's technical conferences known as Brain Share.

WPD Clemson  is working closely with Novell developers to develop a new architecture for NetWare drivers.  NetWare Peripheral Architecture (NPA) drivers developed by Clemson are being used as prototypes for development at Novell.  These drivers will be released when Novell releases this new architecture.

AT&T Global Information Solutions  has contributed equipment to Novell for  use  in  their Super Lab and internal use.  AT&T Global Information Solutions also provides equipment for customer showrooms throughout the US.

AT&T Global Information Solutions is currently investigating these and other areas in which to partner with Novell to provide an even better AT&T Global Information Solutions - Novell relationship which will benefit both companies while delighting our customers.

### Self Certifying Lab

WPD Clemson is now a self-certifying NetWare Lab.  Self-certifying labs must have at least two Certified NetWare Engineers (CNE's) on staff and pass an inspection by Novell.

Testing of hardware and drivers is performed at the Clemson site using test suites provided by Novell.